#

# Ignore stuff coming from ourselves - these IP are ours

#

ip and (not src net 172.16)

and (not src net 192.168)

and

(

(ip[12:4] = ip[16:4])     # land attacks: source ip = dest ip

or

((ip[19] = 0xff) and      # broadcasts to x.x.x.255

not (net 172.16 and udp and (dst port 137 or dst port 138)))

((ip[19] = 0x00) and      # broadcasts to x.x.x.0

not (net 172.16 and udp and (dst port 137 or dst port 138)))

or

#

# fragmented ip packets with more fragments coming and

# fragmented ip packets with zero offset (first in sequence)

#

((ip[6:1] & 0x20 != 0) and (ip[6:2] & 0x1fff = 0))

or

#

#unroutable ip addresses: 0.x.x.x, 127.x.x.x, 1.x.x.x, 2.x.x.x, 5.x.x.x,

#                         240.x.x.x thru 255.x.x.x

#

(net 0)

or

(net 127)

or

(net 1 or net 2 or net 5)

or

(ip[12] > 239)

or

#

# ip packets with options set (header size is bigger than 5 32-bit words)

#

# if want to check for specific options, check ip[20] (given

# that (ip[0:1] & 0x0f > 5) is satisfied):

#

#    option                  ip[20]

#    -------                ---------

#    record route              7

#    timestamp                 0x44

#    loose source routing      0x83

#    strict source routing     0x89

#

( ((ip[0:1] & 0x0f) > 5) )

)