#

# Examine ICMP packets from ourselves.

#

(icmp and

((src net 172.16)

or (src net 192.168.222) or (src net 172.17)

and

(

#

# error messages: fragmentation needed but DF flag set

#

((icmp[0] = 3) and (icmp[1] = 4))

or

#

# error messages: source route failed (look for outbound)

#

((icmp[0] = 3) and (icmp[1] = 5))

or

#

# Request/response for address mask

#

((icmp[0] = 17) or (icmp[0] = 18))

)

)

or

#

# Examine packets coming from outside our network addresses.

#

(icmp and (not src net 172.16)

and (not src net 199.168.222) and (not src net 172.17)

and

(

#

# fragmented icmp packets (look for inbound)

#

(ip[6:1] & 0x20 != 0)

or

#

# Request/response for address mask

#

((icmp[0] = 17) or (icmp[0] = 18))

)

)