CLINK -- Characterize LINKs

---------------------------

Clink is a utility that estimates the latency and bandwidth of

Internet links by sending UDP packets from a single source and

measuring round-trip times.  The basic mechanism is similar to

ping and traceroute, except that clink generally has to send

many more packets.

Clink was written by Allen Downey (downey@colby.edu), based on

pathchar, a similar program written by Van Jaconbson.  The interface

of clink is based on the interface of pathchar, and the underlying

mechanism is based on Jacobson's description of pathchar.  No pathchar

source code is included in clink.

Clink is based on trout, which is a simple version of traceroute

written by Allen Downey, but which is heavily based on the version of

traceroute Richard Stevens presents in his book UNIX Network

Programming: Volume 1, Second Edition.

For Copyright information, see the file named COPYRIGHT that

should have been included with this distribution or contact:

Allen Downey

Colby College

5850 Mayflower Hill

Waterville, ME 04901

downey@colby.edu

http://www.cs.colby.edu/~downey

Information about pathchar and clink:

There is a web page at CAIDA that describes pathchar:

http://www.caida.org/Pathchar/

At this time (19 July 1999), an alpha version of pathchar

is available in binary form for FreeBSD, Linix, Solaris and

some other platforms:

ftp://ftp.ee.lbl.gov/pathchar/

There is also a set of slides there that Jacobson used to

present pathchar at a conference.

In Summer 1998, I wrote the data processing part of clink,

which became process.c.  I used the alpha version of pathchar

to collect data, and wrote a paper describing my experiences,

and proposing some improvements to pathchar's data processing.

That paper will appear at SIGCOMM in September, 1999.  A

Postscript version of the paper is available from

http://www.cs.colby.edu/~downey/pathchar

In Summer 1999, I wrote the data collection part of clink,

called collect.c, and assembled the pieces into this distribution.

I am currently working on (1) implementing the improvements I

suggested in my SIGCOMM paper, and (2) testing additional

improvements.

So far, the primary differences between pathchar and clink are:

1) clink uses the even-odd technique described in the SIGCOMM

paper to generate interval estimates for bandwidth.

2) when clink encounters a routing instability, it collects data

for ALL the paths it encounters, until one of the paths

generates enough data to yield an estimate.

The next major addition to clink will be adaptive data collection

as described in the SIGCOMM paper.

HOW TO COMPILE CLINK

--------------------

To unpack the distribution:

tar -xzf clink.1.0.tar.gz    (if the tar file is compressed)

or

tar -xf clink.1.0.tar        (if the tar file is not compressed)

Move into the newly created directory and compile clink:

cd clink.1.0

make

Clink will probably only compile and run on Linux machines, but

it should be possible to port it to other UNIX platforms without

an enormous amount of work.

Because clink has to open a raw socket, you must either run it as root

or make the executable setuid root.  The latter is preferable; here's

how:

su root

(type the root password)

chmod +s clink

exit

If you try to run clink as a mere mortal, you will get a message like:

clink was unable to open a raw socket.  The most

likely cause is that you are not running it as root.

HOW TO USE CLINK

----------------

Basic invocation

----------------

To characterize the links along a path between you and another host,

type

clink host_name

If clink succeeds, the output will look something like:

clink to host-06.colby.edu (137.146.210.39)

8 probes at each of 93 sizes (28 to 1500 by 16)

0 localhost

|       n=  744  lat= 0.234 ms  bw= (8.493, 8.506, 8.506) Mb/s

1 cb3500-02.switches.network.colby.edu (137.146.194.17)

|       n=  744  lat= 0.182 ms  bw= (9.034, 9.134, 9.552) Mb/s

2 port-0.router-1.network.colby.edu (137.146.238.209)

|       n=  744  lat= 1.020 ms  bw= (4.325, 4.389, 4.458) Mb/s

3 host-06.colby.edu (137.146.210.39)

n is the number of probes that were used to characterize each

link.  In this case, clink makes 8 measurements at each of 93

sizes, for a total of 744 links.  If clink encounters a routing

instability, it may have to send more probes before it gets a

complete set of probes at each size.  If you encounter an alternating

link, you might want to use the -D option to generate a dump

file, and then examine the dump file for more information about

the instability.

lat indicates latency, in milliseconds.  bw indicates bandwidth, in

megabits per second.  Three values are given for bandwidth: a low

estimate, a high estimate, and (in the middle) a best estimate.  The

distance between the high and low estimates gives some indication of

how reliable the estimate is.  For reasons that are explained in the

SIGCOMM paper, the "best" estimate does not necessarily fall between

the high and low values.

Controlling the starting and ending links

-----------------------------------------

If you want to start at something other than the first link,

use the -f option:

clink -f2 host_name

In this case, clink starts with the ttl set to 2, meaning that the

packet will go two hops before returning.  Of course, if the starting

ttl is greater than 1, the estimated characteristics for the

first "link" will really be the aggregate characteristics of the first

few links.

To stop before you get to the destination, use the -m option:

clink -m3 host_name

This example would characterize only the first three links

of the path to host_name.

Controlling the number of probes

--------------------------------

To set the number of probes sent over each link at each size,

use the -q option:

clink -q16 host_name

This example would send 16 probes instead of the default (8).

The -l, -h, and -s option control the range of packet sizes clink

uses, by specifying the low size, the high size, and the step,

respectively.  The default values are 28, 1500 and 16, indicating

sizes from 28 to 1500 by steps of 16 (yielding 93 different values).

These sizes include IP and UDP headers, hence the smallest possible

value is 28 (a UDP packet with no data).  1500 is a common MTU

(maximum transfer unit) for Ethernets.  If the range of sizes spans

the MTU of one of the links, it messes up clink's estimates.

The packet sizes do not include Ethernet headers, which means

that the actual size while the packets are on Ethernet links is

a little bigger (8 bytes bigger, I think).  This omission does

not affect the bandwidth estimates, but it does perturb the latency

estimates of some links just a little.

Controlling the rate of probing

-------------------------------

clink works by sending out a lot of UDP packets that contain

deliberate errors (like a ttl too small to get to the destination or a

port number that is unlikely to be in use at the destination).  These

probes impose a load on the network and on the destination machine.

In fact, sending a barrage of probes to an unsuspecting machine can be

interpreted as an act of hostility.

Users of clink should use caution to avoid overloading networks or

harassing machines at other sites.  By default, clink only sends out

one probe at a time, and it waits a while between probes.

Specifically, it waits for a period equal to 10 times the round trip

time of the previous packet.  Thus, in the worst case, clink-

generated traffic uses 1/11th of the network's capacity.

You can control the wait time between packers with the -r and -i

options.  The -r option changes the ratio between the wait time and

the round trip time.

clink -r12 hostname

This example waits for a period 12 times the round trip time of

each packet before sending the next.

The -i option specifies the intersample time in milliseconds,

independent of the round trip time.

clink -i100 hostname

This example waits 100ms between samples.

Generating dump files

---------------------

The -D option generates a file that contains the raw data

clink collects.  You can specify the name of the dump file.

clink -Dhost.dump host_name

The file host.dump will look like this:

1       137.146.194.17  876     1.303

1       137.146.194.17  1420    1.812

1       137.146.194.17  1340    1.743

1       137.146.194.17  1100    1.505

The first column is the ttl (how many links the packet traversed).

The second column is IP address of the router or host that generated

the ICMP packet.  The third column is the packet size in bytes.  clink

sends the various sizes out in random order.  The fourth column is the

round trip time in milliseconds.

clink also provides an option that prints the SORTT (shortest

observed round trip time) for each packet size sent to each link.

The -M option causes clink to write one file for each link,

with the names hostname.0.mins, hostname.1.mins, etc.  The format

of these file is

28      0.515000

44      0.515000

60      0.529000

76      0.545000

92      0.557000

The first column is the packet size, the second column is the

SORTT in milliseconds.

Reading from a dump file

------------------------

To generate link estimates from a dump file, use the -I option:

clink -Ihost.dump

The output is identical to the output that was generated when

the data was collected.  This feature is probably not useful for

clink users, but it is very useful for trying out variations on

clink's estimation methods.

Avoiding DNS resolution

-----------------------

Normally clinks uses DNS to find the IP address the destination

machine and the names of the intermediate routers.  Because DNS

resolution can be slow (or unavailable), it is sometimes useful

to avoid it.  The -n option forces clink to avoid using DNS.  Of

course, this means that you have to provide the destination machine's

IP address instead of a name.

clink 137.146.210.39

If you are reading data from a dump file, it should be possible

to run clink even if you are not connected to a network.

Verbose mode

------------

The -v flag generates verbose output that looks like this:

clink to host-06.colby.edu (137.146.210.39)

8 probes at each of 93 sizes (28 to 1500 by 16)

0 localhost

sending datagram to port 33437

received ICMP type= 11  code= 0

ttl=1   137.146.194.17  size=  188 B    rtt= 0.832 ms

sending datagram to port 33438

received ICMP type= 11  code= 0

ttl=1   137.146.194.17  size=  780 B    rtt= 1.742 ms

Notice that the destination port gets incremented each time we

send a message.  That makes it possible to check incoming ICMP

messages and make sure they match up with what we sent.  When

the destination port number reaches the maximum (65535) it wraps

back around to 33437.

ICMP type 11, code 0 indicates a time-exceeded error, which

really means that we reached an intermediate router.

ICMP type 3, code 3 indicates a port unreachable error, which

means we have reached the destination.  That's how clink knows

when to stop.

In verbose mode, clink prints messages about all incoming ICMP

packets, including ones that have nothing to do with clink.

kernel-level timestamps

-----------------------

The -k option only works if you have hacked your kernel to put

timestamps in outgoing UDP packets and incoming ICMP packets.  I

did this to my kernel in order to measure the amount of variance

being added by the user-kernel switch, and the effect of that

variance on clink's estimates.

If you use the -k option with an unmodified kernel, you should

get a message like:

I don't think the outgoing kernel timestamps are working

In all likelihood, the incoming stamps aren't working either,

but clink didn't get that far.

If you are interested in the hack I used, contact me (info above).