.   .   .   .   ########################################## .   .   .   .   .

.   .   .   . ##                                      ##   .   .   .   .

.   .   .   .   ##   Async Tools for the Async Minded   ## .   .   .   .   .

.   .   .   . ##   ================================   ##   .   .   .   .

.   .   .   .   ##           Scott A. Deming            ## .   .   .   .   .

.   .   .   . ##          sad@umcc.umich.edu          ##   .   .   .   .

.   .   .   .   ##   No version yet, and no real name.  ## .   .   .   .   .

.   .   .   . ##                                      ##   .   .   .   .

.   .   .   .   ########################################## .   .   .   .   .

============================================================================

----------------------------------------------------------------------------

SETUART.C

=========

void    set_UART_port(int comport, int value);

void    set_UART_int(int comport, int value);

void    set_UART_onmask(int comport, int value);

void    set_UART_offmask(int comport, int value);

Change defaults.  Probably not very useful, but maybe.  No return values,

because there really isn't anything significant here.  These routines

probably wont stick around too long as they seem like they will probably

just clutter the code up.

int     set_parity(int comport, int parity);

Change parity of currently open port.

Returns 1 if successful, 0 if not.

int     set_stopbits(int comport, int stopbits);

Change stop bits of currently open port.

Returns 1 if successful, 0 if not.

int     set_wordlength(int comport, int wordlength);

Change word length of currently open port.

Returns 1 if successful, 0 if not.

int     set_baudrate(int comport, long baudrate);

Change baudrate of currently open port.

Returns 1 if successful, 0 if not.

void    set_handshaking(int comport, int handshaking, int enable);

Set handshaking on comport.  handshaking is one of the HSHAKE

definitions found in ASYNC.H, and enable is 1 to enable, or 0 to

disable.

----------------------------------------------------------------------------

----------------------------------------------------------------------------

DETECT.C -- These routines were taken DIRECTLY from Chris Blums SERIAL port

========    Summary.

int     async_detect_uart(int comport);

Detect what type of UART is on a particular comport.  Returns UART_xxxx

which are MACRO's that can be found in ASYNC.H

int     async_detect_irq(int comport);

Detect what IRQ line is used by a particular comport.  Returns the IRQ

or -1 if it fails for some reason.

----------------------------------------------------------------------------

----------------------------------------------------------------------------

ISR.C

=====

void    init_ISR(int comport);

Initialize the Interrupt Service Routine for a specified comport.

void    deinit_ISR(int comport);

Remove the ISR from comport.

----------------------------------------------------------------------------

----------------------------------------------------------------------------

FIFO.C

======

int     init_fifo(int comport, int enable);

Enable or Disable FIFO on the UART.  Returns 1 if successful, or 0 if not.

It does make sure you have a 16550A before continuing.  This routine will

only work on a port opened with open_port(async.c).

THIS CODE ISN'T COMPLETE YET EITHER.  I JUST HAVEN'T GOTTEN AROUND TO IT YET

----------------------------------------------------------------------------

----------------------------------------------------------------------------

ASYNC.C

=======

int     open_port(int comport, long baudrate, int parity, int stopbits, int wordlength);

Pretty much self explanitory.  If the port is already open, it returns a 0,

otherwise it returns 1 for a successful open.

int     close_port(int comport, int rts, int dtr);

Closes the com port.  If rts != 0 it leaves the rts line the same.  If

dtr != 0 it leaves the dtr line the same.  This is useful for programs

that exit but don't want to hang up the modem.  Returns a 0 if the port

wasn't open to begin with, or a 1 if it closes down successfuly.

----------------------------------------------------------------------------

----------------------------------------------------------------------------

LINE.C

======

int     async_LCR(int comport);

Returns the value of the LCR line on comport.

int     async_MCR(int comport);

Returns the value of the MCR line on comport.

int     async_LSR(int comport);

Returns the value of the LSR line on comport.

int     async_MSR(int comport);

Returns the value of the MSR line on comport.

int     async_DTR_status(int comport);

Returns the status of DTR on comport.

int     async_RTS_status(int comport);

Returns the status of RTS on comport.

void    async_set_DTR(int comport, int status);

Sets the status of DTR on comport.

void    async_set_RTS(int comport, int status);

Sets the status of RTS on comport.

----------------------------------------------------------------------------

----------------------------------------------------------------------------

ASYIO.C

=======

int     async_putch_timeout(int comport, char c, long timeout);

Put a character (c) to comport, failing after timeout milliseconds

Returns 1 on success, and 0 on timeout.

void    async_putch(int comport, char c);

Put a character (c) to comport.

void    async_puts(int comport, char *s);

Put a string (s) to comport.

void    async_printf(int comport, char *fmt, ...);

Printf a string to comport.  Follows the standard printf format.

int     async_putblock_timeout(int comport, const char *block, int size, long timeout);

Put a block (block) of size to comport, failing after timeout milliseconds

Returns 1 on success, and 0 on timeout.

void    async_putblock(int comport, const char *block, int size);

Put a block (block) of size to comport.

int     async_ready(int comport);

Returns 1 if there is a character waiting in the RX buffer of comport,

and 0 if no character is waiting.

char    async_getch(int comport);

Returns the next character in the RX buffer of comport, removing it from

the buffer.

int     async_gets(int comport, char *s, int size)

Gets a string of characters from comport of size bytes, or until a CR

or LF is reached.  Stores the string in s.

Returns the number of characters read from the RX buffer.

int     async_getblock_timeout(int comport, char *block, int size, long timeout);

Pulls up to size bytes from the RX buffer of comport and puts them into

a block (block), removing them from the RX buffer.  This function will

wait for up to timeout milliseconds to meet the size requirements and will

return the actual number of bytes pulled.

int     async_getblock(int comport, char *block, int size);

Pulls up to size bytes from the RX buffer of comport and puts them into

a block (block), removing them from the RX buffer.  Returns the number

of bytes actually pulled from the buffer.

char    async_peek(int comport);

Returns the next character in the RX buffer of comport, leaving it in the

buffer for later retrieval.

void    async_flushrx(int comport);

Flushes the RX buffer on comport.

void    async_flushtx(int comport);

Flushes the TX buffer on comport.

----------------------------------------------------------------------------

MACROs

======

Misc:

NUMBEROFPORTS

Number of COM ports available.  Change this if you add support for any

additional ports via digiboard or whatever.

COM1

COM2

COM3

COM4

COM 1 through 4 MACROS.  Basically just for ease of use.  Note that

COM1 is zero, not one.

Uart:

UART_NONE

UART_8250

UART_16460

UART_16550

UART_16550A

UART identifiers.  These simply tell what UART is detected on the

specified com port with the async_detect_uart() function.

BASE

Base (No real reason to have this)

IER

Interrupt Enable Register

IIR

Interrupt Identification Register

FCR

16550 FIFO Control Register

LCR

Line Control Register

MCR

Modem Control Register

LSR

Line Status Register

MSR

Modem Status Register

SCRATCH

Scratch Pad (Is this ever used?)

Handshaking:

HSHAKE_NONE

HSHAKE_XONXOFF

Use these for setting handshaking on or off with the

set_handshaking() function.

NOTE: These are pretty useless right now, as handshaking is not

supported by this library yet.

Buffers:

TXBUFSIZE

RXBUFSIZE

These are internal MACROs that specify the size of your transmit and

receiving buffers.  Adjust to whatever you'd like.  The memory for

these buffers is allocated dynamically.

Modem Control Register:

MCR_DTR

DTR bit

MCR_RTS

RTS bit

MCR_OUT1

OUT1 bit

MCR_OUT2

OUT2 bit

MCR_LOOPBACK

Loopback bit

Line Control Register:

LCR_PNONE

Parity value -- NONE

LCR_PODD

Parity value -- ODD

LCR_PEVEN

Parity value -- EVEN

LCR_PMARK

Parity value -- MARK

LCR_PSPACE

Parity value -- SPACE

----------------------------------------------------------------------------

Global Variables

================

fifo_enabled

int  : 0 if fifo is disabled, 1 if fifo is enabled.

UART_ports[]

int  : Port values for each com port.

These values can be altered with the set_UART_port() function.

UART_interrupts[]

int  : Interrupt values for each com port.

Can be changed with set_UART_int() function.

UART_onmask[]

int  : Interrupt ON masking for 8259 controller.

Can be changed with set_UART_onmask() function, but probably shouldn't

be.

UART_offmask[]

int  :  Interrupt OFF masking for 8259 controller.

Can be changed with set_UART_offmask() function, but probably shouldn't

be.

struct  async_portS async_port[];

Structure can be found in the file "ASYNC.H"  It's basically the

structure containing all of the information about each specific

serial port.