TURBO C GRAPHICS INFORMATION

����������������������������

This file contains information about the Turbo C graphics that is

not contained in the Additions and Enhancements manual. Please

read this file in its entirety, referring to it when you encounter

problems not addressed in the Additions and Enhancements manual.

TABLE OF CONTENTS

�����������������

1. Adding GRAPHICS.LIB to the Turbo C Libraries

2. Switching to Text Mode

3. Internal Graphics Driver Pointers

4. Graphics Examples on CGA or Hercules

5. IBM 8514 Support

6. Compaq III Portable

7. Loading Drivers and Fonts at Runtime

8. How to Compile BGIDEMO

1. ADDING GRAPHICS.LIB TO THE TURBO C LIBRARIES

�����������������������������������������������

When you use graphics in a Turbo C program you have to either

include graphics.lib on the command line (if using TCC:

"tcc foo.c graphics.lib") or create a project with your program

and graphics.lib in it (TC).  To avoid doing this, you can use

tlib to add the graphics library modules to the standard C*.LIB

libraries.  This may be accomplished easily by one or more of

the following tlib commands (don't leave out the ".lib" extension

on graphics.lib):

tlib cs +graphics.lib                 (small model library)

tlib cc +graphics.lib                 (compact model library)

tlib cm +graphics.lib                 (medium model library)

tlib cl +graphics.lib                 (large model library)

tlib ch +graphics.lib                 (huge model library)

It is probably a good idea to add the graphics modules to all the

standard library files, but if you know you will be using only

some memory models with graphics programs, or you are short on

disk space, you might choose to change only some library files.

Your current directory must be the directory where you keep the C

libraries, or you must include complete paths in the tlib command

line(s). For example:

cd \turboc\lib

tlib cm +graphics.lib

or

tlib \turboc\lib\cm +\turboc\lib\graphics.lib

Note that tlib will rename the original C*.LIB library to C*.BAK;

you might want to delete this file, or keep it around for

reference.

2. SWITCHING TO TEXT MODE

�������������������������

There are two routines that will switch the screen to text mode:

"textmode" and "restorecrtmode". "textmode" should be used only

when the screen is in the text mode (presumably to change to a

different text mode), "restorecrtmode" should be used only when

the screen is in graphics mode (to escape temporarily to text

mode). These are the only contexts where textmode and

restorecrtmode may be used.

3. INTERNAL GRAPHICS DRIVER POINTERS

������������������������������������

There are several far function pointers that are used by the

graphics library to access the low-level graphics routines. These

pointers are set to point to the internal graphics drivers when

"initgraph" is called. Subsequent calls to the graphics routines

will then invoke these internal drivers by calls through the

function pointers. The pointers are not tested for validity (to

handle the case when a graphics routine might be called before

initgraph), as that would introduce a lot of extra code, and slow

down those graphics routines.

In the compact, medium, large, and huge models, these pointers

are, therefore, initialized to point to a routine that will

display an error message, and abort the program (via exit()), in

case a graphics routine is called before initgraph.

In the tiny model there is no way to initialize far pointers to

point to a routine in the code. Furthermore, the tiny model shares

the standard library with the small model. Therefore, the graphics

driver pointers cannot be initialized in the tiny and small

models, and programs that attempt to call a graphics routine

before calling initgraph will most likely crash the machine.

The following graphics routines may be called before initgraph (or

after closegraph): detectgraph, getmoderange, setgraphbufsize,

registerbgidriver and registerfarbgidriver, registerbgifont and

registerfarbgifont.

4. GRAPHICS EXAMPLES ON CGA OR HERCULES

���������������������������������������

Most of the graphics examples in the manual (addendum) will work

best with an EGA or VGA adapter. When run on a CGA or monochrome

adapter, they might not produce the expected results. Therefore,

when running on non-EGA/VGA hardware, use the value "1" (one) in

place of the symbolic color constants, and use the following code

as the example for the arc, circle, ellipse, getarccoords,

getaspectratio, and pieslice routines:

#include <graphics.h>

main()

{

int graphdriver = DETECT, graphmode;

struct arccoordstype    arcinfo;

int     xasp, yasp;

long    xlong;

initgraph(&graphdriver, &graphmode, "");

/* draw a 90 degree arc with radius of 50 */

arc( 100, 120, 0, 89, 50 );

/* get the coordinates of the arc and connect ends */

getarccoords( & arcinfo );

line( arcinfo.xstart, arcinfo.ystart, arcinfo.xend, arcinfo.yend );

/* draw a circle */

circle( 100, 120, 80 );

/* draw an ellipse inside the circle */

ellipse( 100, 120, 0, 359, 80, 20 );

/* draw and fill a pieslice */

setfillstyle( HATCH_FILL, 1 );

pieslice( 200,  50, 0, 134, 49 );

setfillstyle( SLASH_FILL, 1 );

pieslice( 200,  50, 135, 225, 49 );

setfillstyle( WIDEDOT_FILL, 1 );

pieslice( 200,  50, 225, 360, 49 );

/* draw a "square" rectangle */

getaspectratio( & xasp, & yasp );

xlong = ( 50L * (long) yasp ) / (long) xasp;

rectangle( 0, 0, (int) xlong, 50 );

getch();

closegraph();

}

5. IBM-8514 SUPPORT

�������������������

An additional driver has been included with the BGI graphics

library. This driver interfaces to the IBM-8514 graphics card.

This is a new, hi-res graphics card capable of resolutions up to

1024x768 pixels, and a color palette of 256 colors from a list of

256K colors. The driver file name is IBM8514.BGI.

The IBM-8514 graphics card CANNOT be properly autodetected (it will

be recognized as VGA by the autodetection logic). Therefore, to

use the IBM-8514 card, the graphdriver variable must be assigned

the value IBM8514 (which is defined in GRAPHICS.H) when initgraph

is called. Detectgraph, or DETECT with initgraph should not be

used with the IBM-8514 (unless the emulated VGA mode is desired).

The supported modes of the IBM-8514 card are IBM8514LO (640x480

pixels), and IBM8514HI (1024x768 pixels). Both mode constants are

defined in GRAPHICS.H.

The IBM-8514 uses three 6-bit values to define colors. There is a

6-bit Red, Green, and Blue component for each defined color. To

allow the user to define colors for the IBM-8514, a new routine

was added to the BGI library. The routine is prototyped in

GRAPHICS.H as follows:

void far setrgbpalette(int colornum, int red, int green, int blue);

The argument colornum defines the palette entry to be loaded.

colornum is an integer from 0-255 (decimal). The arguments red,

green, and blue define the component colors of the palette entry.

Only the lower byte of these values is used, and out of this byte,

only the 6 most significant bits are loaded in the palette.

The other palette manipulation routines of the graphics library

may not be used with the IBM-8514 driver (i.e., setallpalette,

setpalette, getpalette).

For compatability with the balance of the IBM graphics adapters,

the BGI driver defines the first 16 palette entries of the

IBM-8514 to the default colors of the EGA/VGA. These values can be

used as is, or changed using the setrgbpalette routine.

The floodfill routine will not work with the IBM-8514 driver.

6. COMPAQ III PORTABLE

����������������������

If you are using the Compaq III Portable, enter the following

command at the DOS prompt before trying to use the 400 line mode:

SET SEL=CGA

7. LOADING DRIVERS AND FONTS AT RUNTIME

���������������������������������������

In addition to linking in drivers and fonts (converted by BGIOBJ),

you may load the driver and/or font files directly into memory at

runtime, and then register them through the registerfarbgidriver

and/or registerfarbgifont routines. The following example illustrates

this techique:

/* example of loading a font file into memory */

#include <graphics.h>

#include <io.h>

#include <fcntl.h>

#include <stdio.h>

#include <conio.h>

#include <stdlib.h>

#include <process.h>

#include <alloc.h>

void main()

{

void *   gothic_fontp;  /* points to the font buffer in memory */

int      handle;        /* file handle used for I/O */

unsigned fsize;         /* size of file (and buffer) */

int errorcode;

int graphdriver;

int graphmode;

/* open font file */

handle = open("GOTH.CHR", O_RDONLY|O_BINARY);

if (handle == -1)

{

printf("unable to open font file 'GOTH.CHR'\n");

exit(1);

};

fsize = filelength(handle);           /* find out size of the file */

gothic_fontp = malloc(fsize);         /* allocate buffer */

if (gothic_fontp == NULL)

{

printf("unable to allocate memory for font file 'GOTH.CHR'\n");

exit(1);

};

/* read font into memory */

if (read(handle, gothic_fontp, fsize) != fsize)

{

printf("unable to read font file 'GOTH.CHR'\n");

exit(1);

};

close(handle);                        /* close font file */

/* register font */

if (registerfarbgifont(gothic_fontp) != GOTHIC_FONT)

{

printf("unable to register font file 'GOTH.CHR'\n");

exit(1);

};

/* detect and initialize graphix */

graphdriver = DETECT;

initgraph(&graphdriver, &graphmode, "..");

errorcode = graphresult();

if (errorcode != grOk)

{

printf("graphics error: %s\n",grapherrormsg(errorcode));

exit(1);

};

settextjustify(CENTER_TEXT, CENTER_TEXT);

settextstyle(GOTHIC_FONT, HORIZ_DIR, 4);

outtextxy( getmaxx() / 2, getmaxy() / 2, "Borland Graphics Interface");

getch();                              /* hit a key to terminate */

closegraph();                         /* shut down graphics system */

}

8. HOW TO COMPILE BGIDEMO

�������������������������

With TC.EXE:

1. Run TC.EXE

2. In the Project pulldown menu specify the project name

"BGIDEMO.PRJ"

3. From the main menu select the Run option

With TCC.EXE:

Compile from DOS with the following command line:

TCC bgidemo graphics.lib

Note that BGIDEMO will not run in the tiny model because of that

model's memory limitations.

END

���