PRODUCT SPECIFICATION       COLEX STD-CPU2        6  Aug 1983

(old name  CPUCTC )

Thi� car� i� intende� fo� us� a� th� CP� car� i� bot� minima� �

system� an� dis� base� systems�� I� ha� th� followin� mai� �

features:

-Z80 cpu with 4.0 MHz clock

-6 bytewide sockets

-CTC with buffers and external connections

-8 dynamically selectable memory maps

-MCSYNC output for use with 8088 style bank switching

��������       jumper selectable

-MEMEX output for common memory selection in banked systems

��������       jumper selectable

project engineer:    Andrew Kam

Software support required:               Engineer:   Due:

Install CP/M 3.0

Hardware support required:

$ 500      PC layout                      H+Y Taiwan

$ 500      10 sets parts                  to be ordered

Manpower support:

Technitian/assistant:

Prototype assembly:

Start date: 15 June 1983

Estimated completion:

schematic/parts list       -

1  prototype     w/o SW       -

10 prototypes                 -

100 pre-production             -

production release         -

Production cost/board:

Design cost:

.pa�STD-CPU2 cont'd

The detailed specs of the card are:

-Z80 cpu

- 4.0 Mhz clock

- full STD bus interface

-EPROM/RAM sockets

- 6 sockets

- programmable for 2716/32/64/128/256

6116/6864

48Z02

- accessable from bus with DMA

- 256 x 8 PROM with 8 maps

- MEMEX output to bus

�������� ��������typical uses of MEMEX are:

��������          - force common memory area

��������          - enable alternate memory page

- 'active' signal

- 6 chip selects

��������       - same maps as CPUE

-CTC

- port address 7CH

- with interrupt daisy chain

- buffered inputs/outputs

-MCSYNC

- pulses when port 0FFH is written

- allows bank switching of ram cards

-Dynamically addressable memory maps

- on-board port 0FFH uses bits 0-2 for map selection

###############################################################################################