Release Notes                                          Page 1 of 31

CTOS BMULTI 10.4

November, 1993

CONTENTS                                                    PAGE

Product Interdependencies...................................  2

Restrictions and Known Limitations..........................  7

Special Configuration Options............................... 11

Product Improvements........................................ 13

Migration Guidelines........................................ 21

Support..................................................... 25

Product Information Updates................................. 29

Release Notes                                          Page 2 of 31

PRODUCT INTERDEPENDENCIES

The first three disks of the CTOS BMULTI 10.4 package contains the

following components:

BMULTI Gateway                  10.4

BMULTI SuspendResume Filter     1.3

BMULTIX.25                      1.4

BMULTI Agent                    10.4

The fourth disk is the installation disk for CTOS Printer

PassThrough 2.5.

Release Notes                                          Page 3 of 31

PRODUCT INTERDEPENDENCIES (continued)

One of the following operating system levels is required to run

CTOS BMULTI 10.4:

CTOS III, release 1.1 or higher

CTOS III, release 1.0 or higher

BTOS  II, release 3.0 or higher

CTOS  II, release 3.3 or higher

CTOS   I, release 3.3 or higher

CTOSXE, release 3.0 or higher

XEBTOS, release 7.2, or higher

IDMSS, release 5.1, or higher

Release Notes                                          Page 4 of 31

PRODUCT INTERDEPENDENCIES (continued)

BMULTI 10.4 requires any of the following hardware configurations:

CTOS workstation which includes the XBus, SuperGen Series 5000,

and EISA workstations.

XE520 or XE530 shared resource processor.*

* See information in Migration Guidelines and Coexistance section.

Release Notes                                          Page 5 of 31

PRODUCT INTERDEPENDENCIES (CONTINUED)

In addition, the following optional software may be used with BMULTI:

BTOS X.25 Gateway 9.0 or higher for BMULTIX.25**

IDMSS, release 5.1, or higher

BTOS X.21 Circuit Switching Service 1.2 or higher for X.21

circuit switching

CTOS Modem Service 5.1 or higher

**  BMULTIX.25 users installing or upgrading to X.25 Gateway R10.0,

need to obtain X.25 Gateway s10.0.12 or higher until the the next

release of X.25 becomes available.

Release Notes                                          Page 6 of 31

PRODUCT INTERDEPENDENCIES (CONTINUED)

NOTES:

1)   The BMULTI Gateway is not supported on the SuperGen Series

1000 notebook, or ISA workstations.

2)   Recommended speed for the EISA platforms are as follows (BPS):

Motherboard          IOP                 RS232 expansion module

Sync  = 64k          Sync  = 64k         Sync  = 9600

Async = 19.2k        Async = 38.4k       Async = 9600

Release Notes                                          Page 7 of 31

RESTRICTIONS AND KNOWN LIMITATIONS

BMULTI DMA is not supported on any B39 Series workstation utilizing

the 8274 chip.

Timing problems may occur when using BMULTI with operating systems

prior to CTOS 3.3.

Data compression and error correction capabilities offered on many

of the newer modems may have to be disabled for BMULTI to function

properly.

Release Notes                                          Page 8 of 31

RESTRICTIONS AND KNOWN LIMITATIONS (continued)

Using CTOSXE 3.0.3, or a later operating system, will enhance

performance on the XE530, particularly DMA communications.

The BMULTI Agent configuration file can be corrupted by excessive

saves while using the configurator.  Error 3303 may occur if more

than 25 cumulative saves are performed during a single configuration

session, resulting in the corruption of the configuration file.

When configuring a large number of stations, you can avoid this error

by saving only after every four or five additions, or by saving only

when exiting the configurator, which is done automatically.  If the

configuration file is corrupted, it can be repaired using the CTOS

Maintain File utility.

Release Notes                                          Page 9 of 31

RESTRICTIONS AND KNOWN LIMITATIONS (continued)

A blank station can be accidentally entered from the BMULTI Agent

Configurator, if you press the ADD station function key and do not

enter data before pressing another function key.  You can delete

the blank station entry by moving the cursor to the  blank entry

and pressing the DELETE function key.

Duplicate addresses for different MGS Service IDs are not allowed

by the BMULTI Agent Configurator.

BMULTI is not supported in DMA mode while using channel B of the

host processor as a TDI port on EISA workstations.

Release Notes                                          Page 10 of 31

RESTRICTIONS AND KNOWN LIMITATIONS (continued)

The BmOpenII (request code D02Bh) request in the Request.Sys file

must be modified for NetRouting:

from:  NetRouting:  openFh, devspec, CloseAtTermination

to:    NetRouting:  openFhLL, devspec

Regeneration and installation of the Request.Sys file allows two

BMULTI lines to be installed and communicate on the ID2 module.

Release Notes                                          Page 11 of 31

SPECIAL CONFIGURATION OPTIONS for Printer Pass Through (PPT)

The option :SetBMXlatMode: needs to be added to the PPTConfig0.sys.

The option selects the BMXlatMode used by the BMULTI or BNA.  It

overrides the EnableSISO option.  Valid values are 0,1,2 and 4.

Value:   Explanation:

0        No translation, all characters pass through.  When

transmitting over BMULTI the high order bit of the

extended character is stripped away.

Option 0 is provided for countries that using extended

character sets with BNA or BMX and do not have the same

7 bit protocol restriction as BMULTI.

Release Notes                                          Page 12 of 31

SPECIAL CONFIGURATION OPTIONS for PPT (continued)

Information for the :SetBMXlatMode: values: (continued)

Value:   Explanation:

1        ESC SI indicates ShiftIn state.

ESC SO indicates ShiftOut state.

2        ESC SI or the SI character alone indicates ShiftIn state.

ESC SO or the SO character alone indicates ShiftOut

state.

4        There is no encoding of the data stream.  All characters

will be translated.

Release Notes                                          Page 13 of 31

PRODUCT IMPROVEMENTS

The following provides a summary of the enhancements to BMULTI and

its components in the 10.4 release.

BMULTI 10.4 now runs on EISA workstations.

BMULTI DMA now supports Half Duplex communications.

BMULTI now supports TDI mode on EISA host processors.  To

implement this set the :ECOM: switch to TDI in config.sys and

turn the bluewhite switch on the motherboard to TDI to enable

the TDI drivers.  In addition, set TDI in the BMULTI

configuration file to Yes.

Release Notes                                          Page 14 of 31

PRODUCT IMPROVEMENTS (continued)

Delays can now be reconfigured dynamically without deinstalling

BMULTI.  This addresses problems resulting from insufficient

delay times.  When increasing the RTSHOLD, RTSCTS, and XMTRCV

delays, increase 1 millisecond at a time to avoid adding

unnecessary delays.

This release of CTOS BMULTI supports DMA based communications on

SG5000 workstations.  As in 10.1, BMULTI supports

synchronous and asynchronous communications using DMA.  Baud

rates up to 19.2 kbps are supported with or without DMA in async

mode.  Higher speeds are possible in sync mode.

BMULTI is now available on 3 12 inch release disks.

Release Notes                                          Page 15 of 31

PRODUCT IMPROVEMENTS (continued)

BMULTI LIBRARY

This release resolves several field reported problems in both

the High Level Interface and the Low Level Interface.  To

implement these improvements, relink BMULTI Client

applications with the library provided in the 10.4 release.

Release Notes                                          Page 16 of 31

PRODUCT IMPROVEMENTS (continued)

BMULTIX25

If the X.25 network goes down or there is an interruption on

the X.25 virtual circuit, BMULTIX.25 now handles

reestablishment of the X.25 connection without requiring

any action from the BMULTI Client.  Prior to this release it

often required that the client be deinstalledreinstalled to

be able to reestablish communication.

When an interruption or failure occurs, BMULTIX.25 attempts

to reestablish the connection with the X.25 Gateway every

2 seconds.  After 10 attempts, the time between retries is

extended to 2 minutes.

Release Notes                                          Page 17 of 31

PRODUCT IMPROVEMENTS (continued)

BMULTIX25

BMULTIX.25 will accept any action by the clients when the X.25

connection is unavailable.  BMULTI notifies the client by a

transmit or receive error, of any potential data loss if the

failure occured during a transmit or receive.  If transmitting,

BMULTIX.25 will keep the BMULTI Client in transmit until the

connection is reestablished and the message can be sent (which

may be up to 2 minutes after the circuit comes back).  It

is the responsibility of the client to abort the transmission

if it cannot wait. The behavior is similiar to that of BMULTI

when the pollselect line is not available.

Release Notes                                          Page 18 of 31

PRODUCT IMPROVEMENTS (continued)

LOGGING

Logging to the BMX25.log is now optional.  The default is no

logging.  Any fatal errors are now logged to the system plog.

The command case is used to identify the logging option.

CASE 00 (default) No Logging

01           Unlimited Logging (previous releases)

If using the command rather than the JCL to install, either

use the run command to initiate BMX25Zip.run or edit the

command to change the case.

Release Notes                                          Page 19 of 31

PRODUCT IMPROVEMENTS (continued)

LOGGING (continued)

Only use logging if necessary when troubleshooting.  If used,

it should be saved off regularly and deleted.

When an error is received by the BMULTI Agent from the BMULTI

Gateway during retrieval of receive data, an entry is made to

PLOG, and a message is sent to MCS for logging.  The host station

returns to Receive Ready state and will be able to continue

transmitting.  Data received should be verified if this error

occurs.

Release Notes                                          Page 20 of 31

PRODUCT IMPROVEMENTS (continued)

BMULTI AGENT

The maximum size message from the host that the BMULTI Agent

packaged with this release can receive is 4096 bytes. Also note

that the Message Control Module (MCM) maximum buffer size should

be equal or greater than that of BMULTI.

The BMULTI Agent can now be configured to issue a Detach as well

as a Not Ready The Station when the line goes down.  This

restores the functionality available in BMULTI Agent R10.0.03.

To have the agent issue a Detach, set the installation parameter

Detach On Tx Error (Default=y) to D.

Release Notes                                          Page 21 of 31

MIGRATION GUIDELINES

CTOS BMULTI 10.4 is an upgrade release with minor enhancements.

Follow the installation process.  The BMULTI request codes have

been updated (see Product Improvements) and are now at version 4.

Execute the BMULTI Configurator to update the configuration file.

Failure to update configuration files may cause unpredictable

results.

BMULTI can coexist with Context Manager II, BNet II, and the

Multiple Gateway Server (MGS).  Old configuration files, however,

must be updated if you want to use them with this release.

Failure to update configuration files may cause unpredictable

results.

Release Notes                                          Page 22 of 31

MIGRATION GUIDELINES (continued)

The following information is relevant if upgrading from BMULTI

release prior to BMULTI 10.2.

In order to accommodate newer modems, BMULTI now provides

faster line turn around.  BMULTI can now handle RS232toTDI

converters and modems with shorter TransmitReceive (XMTRCV),

RequesttoSend(RTS)CleartoSend(CTS), and RTSHOLD delays.

Previously, BMULTI added delay time to compensate for slower

devices.  BMULTI no longer adds delay time, therefore you may

need to increase your configuration values for specific devices

to achieve the same delay.

Release Notes                                          Page 23 of 31

MIGRATION GUIDELINES (continued)

The following information is related to running BMULTI 10.4 on

shared resource processors or EISA workstations:

BMULTI request codes default to accomodate one copy of BMULTI in

real memory on a shared resource processor or EISA workstation.

To install multiple copies of BMULTI( that is, one on each

processor), set the :SRPRouting: switch for all BMULTI requests

to 'rDevice,' except DeInstall Request which must be local.

To install multiple copies of BMULTI on the same board using

Multiple Gateway Server (MGS) set the SRPRouting: switch for

all MGS requests to rLocal.  All BMULTI requests should be

rRemote except deinstallation, which should be rLocal.

Release Notes                                          Page 24 of 31

MIGRATION GUIDELINES (continued)

To avoid future installation conflicts, increase the version

number of the request files being updated.  Please refer to

the Operating Systems Concepts Manual for more information on

request codes for shared resource processors and EISA

workstations.

Release Notes                                          Page 25 of 31

SUPPORT

Warranty

CTOS BMULTI 10.4 is a warranted Unisys software product.

Unisys warrants that CTOS BMULTI 10.4, in its unaltered form,

will substantially conform to Unisys current published functional

specifications when used in Unisys supported configurations.  The

customer is responsible for reporting suspected deviations using

the User Communication Form (UCF) process within ninety (90) days

following delivery.  The product is neither field repairable nor

field modifiable.  Corrections to reported deviations are provided

to the user via updated code files.

Release Notes                                          Page 26 of 31

SUPPORT (continued)

This standard warranty is not a substitute for Unisys Service

Agreement offerings.

Support Category

CTOS BMULTI 10.4 is fully supported by Unisys.

Support services for CTOS BMULTI 10.4 are offered under the

SURETY (United States) or A la Carte (International) support

programs and are consistent with the support services offered for

the entire CTOS Hardware and Software family of products.

Release Notes                                          Page 27 of 31

SUPPORT (continued)

Unisys SureNet is an electronic bulletin board service, providing

customers in the United States access to technical information seven

days a week, 24 hours a day.  SureNet Technical Information Services

are available at no additional cost to customers with a current

Unisys service agreement or software license and include access to

system alerts, customer technical bulletins and online UCF entry.

Registration is required.  SureNet Support Services are available on

an annual subscription basis.  For more information, contact Unisys

Direct at 18004481424, prompt 5.  You may register for Technical

Information Services andor Support Services either online (by

dialing 18008288796, 8 data bits, 1 stop bit, no parity,

asynchronous, VT100 emulation) or by calling Unisys Direct.

Release Notes                                          Page 28 of 31

SUPPORT (continued)

If you need further assistance or information regarding support

services, please contact your Unisys representative or your local

Unisys office.

Support Discontinuance

Unisys supports the current release level and one level back.

Prior levels are not supported.

Release Notes                                          Page 29 of 31

PRODUCT INFORMATION UPDATES

CTOS BMULTI Operations and Programming Guide (3915 4133000)

*** page 12 ***

Add the following to the information on procedural interfaces.

Both the interfaces allow a userwritten application to open a

session, communicate on the session and terminate the session.

The interfaces support up to a maximum of five different

sessions simultaneously. This means that a BMULTI application

cannot have more than five active sessionsfive active

addresses at any time.

Release Notes                                          Page 30 of 31

PRODUCT INFORMATION UPDATES (continued)

If any user application needs to have more than five active sessions,

different application run files, each with a maximum of five active

stations, must be generated.  Through the use of standard CTOS

software procedures, these run files can be tied together to make

one application.

Release Notes                                          Page 31 of 31

PRODUCT INFORMATION UPDATES (continued)

*** page 410 ***

The maximum communications speed for asynchonous communications on

the XE530 using DMA is 19,200 and not 38,400 as stated in the

manual.