STANFORD ETHERNET

PACKET FILTER

FOR 4.2BSD UNIX

HOW TO ADD A NEW DEVICE

JEFFREY MOGUL

COMPUTER SCIENCE DEPARTMENT

STANFORD UNIVERSITY

8 AUGUST 1984

INTRODUCTION

The  ethernet  packet filter is a pseudo-device driver that uses actual 4.2BSD

network drivers to send and receive packets.   There  is  a  relatively  clean

interface  between  the packet filter and the drivers.  The packet filter code

itself is device-independent; it should also be  machine-independent  (but  we

have not tested this.)  This document describes the interface and discusses an

actual example.

OVERVIEW: THE PACKET FILTER

The  user view of the packet filter is described in the associated unix manual

page (enet(4)).  Briefly, it demultiplexes each ethernet interface, or similar

device, into a fixed number  of  minor  devices  (accessible  via  entries  in

/dev/.)    Using  a  set of ioctls, a program can arrange to receive a more or

less arbitrary set of packets; it may also send any packet.  The packet filter

provides a raw interface to the network; it does not add  or  remove  headers,

nor  does  it  modify  packets  (except to insert the proper source address on

transmission.)

Internally, the packet filter maintains an  incoming  packet  queue  for  each

minor device; packets arriving from the network drivers are distributed to the

appropriate  queues.    Outgoing  packets are simply handed to the appropriate

network driver.

INTERFACE BETWEEN PACKET FILTER AND DRIVERS

The packet filter has three points of contact with  a  network  driver.    One

entry  is  called by the network driver during its ``attach'' phase; the order

in which interfaces are attached determines their  ``unit  number'',  a  small

integer  that the driver will later use to tell the packet which unit a packet

was received on.  Another entry into the packet filter is used for the  driver

to  hand  a  received packet to the filter.  The packet filter, in turn, calls

the driver's ``output'' entry to send a packet.

Attaching:

unit_number = enetattach(ifp, devp)

int unit_number;

struct ifnet *ifp;

2

struct endevp *devp;

When  a driver ``attaches'' to the packet filter, it passes the address of two

data structures: the associated struct ifnet that describes the interface  and

its  driver, and a struct endevp, a structure that describes attributes of the

interface that are interesting to user-level  programs.    The  packet  filter

stores  the address of the struct ifnet, but makes a copy of the struct endevp

to simplify the driver's storage  management.    The  value  returned  is  the

assigned ``unit number''.

Receiving:

enetFilter(unit_number, mtop, length)

struct mbuf *mtop;

int length;

When  a  packet  is  received  that  the  driver does not have to deliver to a

specific protocol (e.g., IP or ARP), it should hand it to the  packet  filter.

This  routine  takes  three  arguments:  the  appropriate ``unit number'', the

address of the first mbuf of the entire packet, including the data-link  layer

header, and the length in bytes of the entire packet.

Transmitting:  When  the  packet  filter  sends  a packet, it simply calls the

driver's ``output''  routine  in  the  normal  way.    However,  it  passes  a

``destination  address''  of family AF_IMPLINK, to indicate to the driver that

the local net header has already been pre-pended to the packet.    The  driver

should  fill in the packet source address, but should not otherwise modify the

packet.

EXAMPLE/COOKBOOK

The rest of this document shows the changes necessary to use the packet filter

with the Interlan 10mb Ethernet driver,  vaxif/if_il.c,  as  distributed  with

4.2BSD   in  late  1983  (subsequent  versions  of  the  Interlan  driver  are

cosmetically different; this should not materially affect the example.)

Before continuing, you should read and understand  the  manual  page  for  the

packet  filter (enet(4)).  It might also be a good idea to read and understand

the code in these three files:

vaxif/enet.h

vaxif/enetdefs.h

vaxif/enet.c

You should also understand the code for the driver you are modifying.

(1.) Add references to the appropriate header files. enetfilter.h lives in the

configuration-specific directory (it is created by /etc/config).    This  file

defines  NENETFILTER.    vaxif/enet.h  is  the  header file for linkage to the

packet filter code.

***************

3

*** 1,6

/*    if_il.c 6.2     83/09/24        */

#include "il.h"

/*

* Interlan Ethernet Communications Controller interface

--- 1,7 -----

/*    if_il.c 6.2     83/09/24        */

#include "il.h"

+ #include "enetfilter.h"

/*

***************

*** 32,37

#include "../vaxif/if_il.h"

#include "../vaxif/if_ilreg.h"

#include "../vaxif/if_uba.h"

#include "../vaxuba/ubareg.h"

#include "../vaxuba/ubavar.h"

--- 35,43 -----

#include "../vaxif/if_il.h"

#include "../vaxif/if_ilreg.h"

#include "../vaxif/if_uba.h"

+ #if   NENETFILTER > 0

+ #include "../vaxif/enet.h"

+ #endif        NENETFILTER > 0

#include "../vaxuba/ubareg.h"

#include "../vaxuba/ubavar.h"

(2.)  Add a field to the software status structure (in this case, il_softc) to

remember which enet unit number this interface maps to.

***************

*** 70,75

struct  il_stats is_stats;      /* holds on-board statistics */

struct  il_stats is_sum;        /* summation over time */

int     is_ubaddr;              /* mapping registers of is_stats

*/

} il_softc[NIL];

ilprobe(reg)

--- 77,89 -----

struct  il_stats is_stats;      /* holds on-board statistics */

struct  il_stats is_sum;        /* summation over time */

4

int     is_ubaddr;              /* mapping registers of is_stats

*/

+ #if   NENETFILTER > 0

+       short   is_enetunit;            /* unit number for enet

filtering */

+ #endif        NENETFILTER > 0

} il_softc[NIL];

ilprobe(reg)

(3.)  In the ``attach'' routine (in this case, ilattach()), add code to attach

this interface to the packet filter.  This involves setting up a struct endevp

to store device-specific information.  The address of this structure, together

with the address of the interface's struct ifnet, are passed to  enetattach();

the  value  returned  is  the  unit number to be stored in the software status

structure.

***************

*** 153,158

#ifdef notdef

is->is_ifuba.ifu_flags |= UBA_NEEDBDP;

#endif

if_attach(ifp);

}

--- 167,188 -----

#ifdef notdef

is->is_ifuba.ifu_flags |= UBA_NEEDBDP;

#endif

+ #if NENETFILTER > 0

+       {

+           struct endevp enp;

+

+           enp.end_dev_type = ENDT_10MB;

+           enp.end_addr_len = sizeof(is->is_addr);

+           enp.end_hdr_len = sizeof(struct ether_header);

+           enp.end_MTU = ETHERMTU;

+           bcopy((caddr_t)(is->is_addr),

+                       (caddr_t)(enp.end_addr), sizeof(is->is_addr));

+           bcopy((caddr_t)etherbroadcastaddr,

+                       (caddr_t)(enp.end_broadaddr),

sizeof(is->is_addr));

+

+           is->is_enetunit = enetattach(&is->is_if, &enp);

+       }

+ #endif NENETFILTER > 0

if_attach(ifp);

(4.) Modify the receiver interrupt routine (ilrint,  in  this  case)  to  pass

otherwise   unclaimed   packets   to   the   packet  filter.    This  involves

reconstructing the data-link layer header that was previously thrown away, and

5

linking this mbuf in front of the rest of the packet (now also in mbufs).  The

mbuf  chain,  together  with  the unit number and total packet length, is then

passed to the enetFilter routine (which wil  take  care  of  deallocating  the

mbufs.)

***************

*** 429,434

goto setup;

#endif

default:

m_freem(m);

goto setup;

}

--- 473,513 -----

#endif

+ #if NENETFILTER > 0

default:

+       {

+               register struct mbuf *mtop;

+               /*

+                * We need the local net header after all.

+                */

+               MGET(mtop, M_DONTWAIT, MT_DATA);

+               if (mtop == 0) {        /* no more mbufs? */

+                       m_freem(m);             /* wasted effort */

+                       goto setup;

+               }

+               il->ilr_type = htons((u_short)il->ilr_type);

+                       /* undo the swap done earlier */

+

+                       /*

+                        * the struct il_rheader pointed to by "il"

+                        * contains the received ether header, but

+                        * the definitions should be more obvious.

+                        */

+               bcopy((struct ether_header*)il->ilr_dhost,

+                       mtod(mtop, struct ether_header *),

+                       sizeof(struct ether_header));

+               mtop->m_len = sizeof(struct ether_header);

+               mtop->m_next = m;

+               enetFilter(is->is_enetunit, mtop,

+                               (len + sizeof(struct ether_header)) );

+               goto setup;

+       }

+ #else

+       default:

m_freem(m);

goto setup;

+ #endif NENETFILTER > 0

Note: some devices require more complex code here.  For example, the 3com 10mb

6

ethernet  does  not  do  DMA  and  thus  the  Vax  processors  must access the

interface's buffers via the UBA; this only supports 16-bit operations, so  the

bcopy()s shown about must be changed to explicit loops.

(5.)  Modify  the  output  routine (in this case, iloutput to allow the packet

filter to send packets with user-specified headers; the driver still fills  in

the  source  address  but  does not need to allocate space for the header, nor

should it modify the other fields.

***************

*** 501,506

goto gottype;

#endif

case AF_UNSPEC:

il = (struct ether_header *)dst->sa_data;

bcopy((caddr_t)il->ether_dhost, (caddr_t)edst, sizeof

(edst));

--- 582,593 -----

goto gottype;

#endif

+

+       case AF_IMPLINK:

+               il = mtod(m, struct ether_header *);

+               goto gotheader;

+

case AF_UNSPEC:

il = (struct ether_header *)dst->sa_data;

bcopy((caddr_t)il->ether_dhost, (caddr_t)edst, sizeof

(edst));

***************

*** 548,553

il = mtod(m, struct ether_header *);

il->ether_type = htons((u_short)type);

bcopy((caddr_t)edst, (caddr_t)il->ether_dhost, sizeof (edst));

bcopy((caddr_t)is->is_addr, (caddr_t)il->ether_shost, 6);

/*

--- 635,642 -----

il = mtod(m, struct ether_header *);

il->ether_type = htons((u_short)type);

bcopy((caddr_t)edst, (caddr_t)il->ether_dhost, sizeof (edst));

+

+ gotheader:

bcopy((caddr_t)is->is_addr, (caddr_t)il->ether_shost, 6);

/*