Note: References to Stargate are specific to that multi serial port board.

References to the 8250 apply to all 8250 UART chips. Some comments may

indicate that a specific condition is needed a a specific serial board.

Below are some UNTESTED routines which I believe will help in setting

some com parameters. These routines have not reached the debug stage,

so no gaurantees. I was originally writting this for a complex system

which would address several serial port locations, port[port_num] in the

routines below refer to the base port address of the specific 8250 chip.

That project has been put on hold at the present time. I present this

information to the BBS comunity because recent programs have produced

8250 transfer rates at speeds of 115K+ baud, much above documented

transfer rates which are deemed realistic. These programs appear to function

without any adverse conditions at these rates. So, I hope all hackers can

delight in the information which follows.

These routines were written in TURBOC Ver 1.5 and are given only to show

how I believe setting the parameters could be done. Again, These routines

have not been tested or debugged.

STARGATE OC8000 SETTINGS

PORT ADDRESSES      HEX LOCATION 1      HEX LOCATION 2

PORT 1                   180                 3F8  (Normally COM1)

PORT 2                   188                 2F8  (Normally COM2)

PORT 3                   190                 280

PORT 4                   198                 288

PORT 5                   1A0                 290

PORT 6                   1A8                 298

PORT 7                   1B0                 2A0

PORT 8                   1B8                 2A8

Interrupt

Status Register          580                 7F8

HARDWARE INTERRUPT SET TO IRQ3 (used by COM2 devices) OR

HARDWARE INTERRUPT SET TO IRQ4 (used by COM1 devices) OR

HARDWARE INTERRUPT IRQ2 MAY BE USED IF IT IS NOT TAKEN BY ANOTHER

DEVICE, THUS PRESERVING COM1,COM2 PORTS AS NORMAL.

HIGHER IRQ SETTINGS NOT RECOMMENDED SINCE THEY HAVE LOWER PRIORITIES

AND/OR MAY BE TAKEN BY OTHER DEVICES. A LIST OF HARDWARE IRQ

ASSIGNMENTS IN DOS ARE SHOWN BELOW.

Switch Settings on OC8000 board:

( Set one of the switches 1-6 to on to set IRQ# )

( Lowest IRQ has highest interrupt priority.    )

Dip Switch 1   Location 1          Location 2     IRQ

8    OFF                 ON

7    ON                  ON             SEE DOC

(--------- CHOOSE ONE ONLY --------)

6    ON SETS OFF DISABLES IRQ           7

5    ON SETS OFF DISABLES IRQ           6

4    ON SETS OFF DISABLES IRQ           5

3    ON SETS OFF DISABLES IRQ           4

2    ON SETS OFF DISABLES IRQ           3

1    ON SETS OFF DISABLES IRQ           2

Dip Switch 2 sets number of uarts in last 4 positions.

OC 8000 board with # UARTS;         DS2 SWITCH POS.

UART LOCATION      NUM             1    2    3    4

4              ON   ON   ON   ON

TOP                 5              OFF  ON   ON   ON

SECOND FROM TOP     6              OFF  OFF  ON   ON

SECOND FROM BOTTOM  7              OFF  OFF  OFF  ON

BOTTOM              8              OFF  OFF  OFF  OFF

In looking at the OC8000 board, these switches appear to fix pin

30 of the 8250 chip to ground. This is the interrupt pin that goes

high when any of the allowed IER interrupts occur on the 8250.

Grounding this pin effectively disables any hardware interrupts

being sent to the CPU via the Stargate Interrupt Status Register.

-------------------------------------------------------------------------------

== General IBM Hardware interrupt locations. PC-XT and AT ==

Hardware Interrupt Vector memory locations and number: DOS 2.X - 3.X

IRQ                                   Number

(hex) - (Decimal)

IRQ0 System Timer Tick Attention - 0x20    - 8

IRQ1 Keyboard Attention          - 0x24    - 9

IRQ2 Reserved for future use     - 0x28    - 10

IRQ3 Com2 Attention              - 0x2C    - 11

IRQ4 Com1 Attention              - 0x30    - 12

IRQ5 Harddisk Attention          - 0x34    - 13

IRQ6 Disk controller Attention   - 0x38    - 14

IRQ7 Parallel Printer Attention  - 0x3C    - 15

-------------------------------------------------------------------------------

===============================================================================

Detailed 8250A register locations for performing IO.

BASE+0    TX CHARACTER BUFFER      if DLAB=0

--------------------------------------------------------------

BASE+0    LSB DIVSOR LATCH         if DLAB=1

--------------------------------------------------------------

BASE+0    RX CHARACTER BUFFER      if DLAB=0

--------------------------------------------------------------

BASE+1    MSB DIVSOR LATCH         if DLAB=1

--------------------------------------------------------------

BASE+1    INTERRUPT ENABLE REG     if DLAB=0 (IER)

ENABLES FOUR 8250 INTERRUPT TYPES TO SEPERATELY ACTIVATE

THE CHIP (INTRPT) OUTPUT SIGNAL. A 0 IN BITS 0-3 DEACTIVATE

ALL INTERRUPTS, AND PLACING A 1 IN ANY BIT 0-3 ACTIVATES

THE INTERRUPT.

D7=0

D6=0

D5=0

D4=0

D3= ENABLE MODEM STATUS INTERRUPT

D2= ENABLE RECEIVE LINE STATUS INTERRUPT

D1= ENABLE TX HOLDING REGISTER INTERRUPT

D0= ENABLE RECEIVE DATA AVAILABLE INTERRUPT

--------------------------------------------------------------

BASE+2    INTERRUPT ID REGISTER    (IIR)

D7=0

D6=0

D5=0

D4=0

D3=0

D2-D1     INDICATES THE HIGHEST 8250A INTERNAL INTERRUPT

PENDING. SUMMARIZED BELOW.

D0=       0=INDICATES INTERRUPT PENDING.

1=INDICATES NO INTERRUPT PENDING.

BIT 2 1 0 PRIORITY  INTERRUPT           INTERRUPT      RESET

TYPE                SOURCE         CONTROL

--------------------------------------------------------------

1 1 0  HIGHEST  RX LINE STATUS      OVERRUN,       READ

PARITY,        LINE

FRAMEING       STATUS

ERRORS,        REGISTER

OR BREAK

INTERRUPT

--------------------------------------------------------------

1 0 0 SECOND    RECEIVED            RECEIVER       READ

DATA                DATA           RECEIVER

AVAILABLE           READY          BUFFER

REGISTER

--------------------------------------------------------------

0 1 0 THIRD     TX HOLDING          TX HOLDING     READ

REGISTER IS         REGISTER IS    (IIR) IF

EMPTY               EMPTY          SOURCE OF

INTERRUPT

OR

WRITE TO

TX HOLDING

REGISTER

-----------------------------------------------------------------

0 0 0 FOURTH    MODEM STATUS        CTS,DSR,       READ

RING           MODEM

INDICATOR,     STATUS

OR RECEIVE     REGISTER

LINE SIGNAL

DETECT

--------------------------------------------------------------

BASE+3    LINE CONTROL REG         (LCR)

DLAB state=bit 7

D7;  DLAB

D6;  0=break disabled.

1=break  enabled.

D5;  0=parity disabled.

1=parity enabled.

D4;  0=odd parity.

1=even parity.

D3;  0=no parity.

1=parity.

D2;  0=1 stop bit.

1=2 stop bits if bits/char > 5, else 1.5 stop bits.

D1-0;00=5 bits/char

01=6 bits/char

10=7 bits/char

11=8 bits/char

--------------------------------------------------------------

BASE+4    MODEM CONTROL REG        (MCR)

IMPORTANT:

Bit 3 must be 0 to allow for IRQ in the Interrupt Enable Reg.

to take place.

D7=0;

D6=0;

D5=0;

D4;  Loopback for internal testing of the 8250 chip.

In diagnostic mode when set to 1;

Diagnostics off when reset to 0 (Normal Operation);

D3;  Controls User Designated OUT2 signal output.

(8250 UART PIN 34)

(Not used on Stargate Card OC8000)

(Must be 0 to allow IER interrupts on IBM cards,

don't know about others.)

When set to 1, OUT2 output forced to logic 0.

When reset to 0, OUT2 output forced to logic 1.

D2;  Controls User Designated OUT1 signal output.

(8250 UART PIN 34)

(Not used on Stargate Card OC8000)

When set to 1, OUT1 output forced to logic 0.

When reset to 0, OUT1 output forced to logic 1.

D1;  Controls RTS output.

(8250 UART PIN 32, RS232C PIN 4)

(Available on Stargate OC8000 Port 1 only)

When set to 1, RTS output forced to logic 0.

When reset to 0, RTS output forced to logic 1.

D0;     Controls DTR output.

(8250 UART PIN 33, RS232C PIN 20)

(Available on Stargate OC8000 Port 1 only)

When set to 1, DTR output forced to logic 0.

When reset to 0, DTR output forced to logic 1.

--------------------------------------------------------------

BASE+5    LINE STATUS REG          (LSR)     LOGIC STATES;

D7=0

D6=TX SHIFT REG EMPTY(TSRE)        1=TSRE IDLE

0=DATA TRANSFER FROM

THRE TO TSRE.

D5=TX HOLDING REG EMPTY (THRE)     READY TO EXCEPT NEW CHAR

1=ISSUES IRQ TO CPU, SET

WHEN CHAR IS TRANSFERRED

FROM THRE TO TSRE.

0=RESET WHEN THRE LOADED

FROM CPU.

(Bits 1-4 are error conditions that produce a Receiver

Line Status Interrupt when their conditions are detected)

D4=BREAK INTERRUPT (BI)            1=RECEIVED DATA HELD LONGER

THAN SPACING STATE(Logic 0)

FOR LONGER THAN FULL WORD

TRANSMIT TIME.

D3=FRAMING ERROR (FE)              1=INDICATES INVALID STOP

BIT, STOP BIT DETECTED AS

PIN 31 8250A                     0 (SPACING LEVEL) FOLLOWING

LAST DATA OR PARITY BIT.

D2=PARITY ERROR (PE)               1=INDICATES WRONG PARITY AS

SELECTED IN LINE CONTROL

PIN 34 8250A                     REG. RESET BY HAVING CPU

READ LINE STATUS REGISTER.

D1=OVERRUN ERROR (OE)              1=DATA IN RECEIVE BUFFER REG

RTS PIN 4 RS232                      NOT READ BY CPU BEFORE NEXT

PIN 32 8250A                     CHAR TRANSFERRED INTO THE

RECEIVE BUFFER REG, THUS

DESTROYING PREVIOUS CONTENTS.

RESET BY READING LINE STATUS

REGISTER.

D0=DATA READY (DR)                 1=INDICATES A COMPLETE CHARACTER

DTR PIN 20 RS232                     HAS BEEN RECEIVED AND PLACED

PIN 33 8250A                     INTO THE RECEIVE BUFFER REG.

RESET BY READING (RBR) OR

WRITING A 0 TO THIS BIT BY CPU.

--------------------------------------------------------------

PINS ON - RS232    8250A

BASE+6    MODEM SATUS REG          (MSR)

D7= (RLSD) RECEIVE LINE SIGNAL DETECT             8

D6= (RI) RING INDICATOR                           22

D5= (DSR) DATA SET READY                          6

D4= (CTS) CLEAR TO SEND                           5

(The bits below are 1 if state of bits above changed since

the last reading of this register.)

D3= (DRLSD) DELTA RECIEVE LINE SIGNAL DETECT                38

D2= (TERI) TRAILING EDGE RING INDICATOR                     39

D1= (DDSR) DELTA DATA SET READY                             37

D0= (DCTS) DELTA CLEAR TO SEND                              36

------------------------------------------------------------------------

Summary of 8250A register locations for performing IO.

BASE+0    TX CHARACTER BUFFER      if DLAB=0

BASE+0    LSB DIVSOR LATCH         if DLAB=1

BASE+0    RX CHARACTER BUFFER      if DLAB=0

BASE+1    MSB DIVSOR LATCH         if DLAB=1

BASE+1    INTERRUPT ENABLE REG     if DLAB=0 (IER)

BASE+2    INTERRUPT ID REGISTER    (IIR)

BASE+3    LINE CONTROL REG         (LCR)

BASE+4    MODEM CONTROL REG        (MCR)

BASE+5    LINE STATUS REG          (LSR)

BASE+6    MODEM SATUS REG          (MSR)

------------------------------------------------------------------------

NOTE: Very little information was found on the 8259 interrupt controller

chip. What was found and could be verified to the best of my

knowledge is given below.

8259 Interrupt Controller Notes:

Acoording to research information,

Port Location 0x20 MUST BE SENT A 0x20 TO INDICATE AN EOI.

However in a review of BIOS async routines, no such EOI is sent.

It is used however to indicate a state change on the keyboard

interrupt BIOS routine.

Port Location 0x21 is the IRQ0-IRQ7 Hardware Interrupt Mask.

0=Interrupt Enabled, 1=Interrupt Disabled.

**************************************************************************/

#define SET_DLAB1 outportb(port[port_num]+3,(0x80|inport(port[port_num]+3)))

#define SET_DLAB0 outportb(port[port_num]+3,(0x7F&inport(port[port_num]+3)))

#define CLOCK_SPEED 1843200

/**** Set Baud Rate

void set_baud (short int port_num)

Sets the baud rate of an 8250 chip at the base address

on port_num.

Buad rate errors are minimal at baud rates <= 9600.

****/

void set_baud (short int port_num)

{

unsigned short int divisor;

divisor=CLOCK_SPEED/(baud[port_num]*16);

SET_DLAB1;

outport(port[port_num],divisor);

SET_DLAB0;

return;

} /* end set_baud() */

/**** Set frameing parameters on communications port.

void set_frame(short int port_num)

Sets the parity, bits/character, and number of stop bits.

port_num is used to get pointer to base address of 8250 chip.

Meaning of frame bits.

D7;  DLAB

D6;  0=break disabled.

1=break  enabled.

D5;  0=parity disabled.

1=parity enabled.

D4;  0=odd parity.

1=even parity.

D3;  0=no parity.

1=parity.

D2;  0=1 stop bit.

1=2 stop bits if bits/char > 5, else 1.5 stop bits.

D1-0;00=5 bits/char

01=6 bits/char

10=7 bits/char

11=8 bits/char

****/

void set_frame(short int port_num)

{

unsigned char f_byte,temp;

int i;

SET_DLAB0;

f_byte=0;

/* set parity */

if (parity[port_num] =='E' | parity[port_num]=='e') f_byte=0x3F;

if (parity[port_num] =='O' | parity[port_num]=='o') f_byte=0x2F;

/* set word length */

temp=wordlen[port_num]-5;

f_byte |=temp;

/* set stop bits */

if(!(2-stopbits[port_num])) f_byte |='\04';

/* configure port */

outportb(port[port_num]+3,f_byte);

return;

} /* end set_frame() */

########################################################################################################