Date: 11 April 1982 00:53-EST

From: Devon S. McCullough

Subject:  Apple Disk drive frying

To: All

Fried  Apple drives are so common that it's a non-warranty repair

which is supposed to cost $140 to fix.  It's too easy to plug 'em

in wrong.  Also, the new RFI-shielded cables are too short, makes

it  easy  to  yank the cable (their strain  relief  doesn't  work

anyway) when the drives are moved,  and makes it likely you  will

move  'em accidentally,  which can and will lead to drive-frying.

One  guy  in  the DC area here got an Apple in  the  morning  and

returned  to the shop after lunch with that  telltale  discolored

paint  on the drive cabinet...fortunately the dealer took pity on

him and gave him a new unit, taking the cooked one back.

Date: 12 Apr 1982 1007-EST

From: Chuck Perilli

Subject: Re: Apple Disk drive frying

To: All

95%  of  the time,  the only problem with a cooked drive  is  the

74LS125 chip.  The gotcha is that sometimes the socket melts too,

often requiring replacement.

Apple has come out with a longer RFI drive cable.  I have seen it

on  some  of the new drives.   Dealers might be able  to  special

order it.

#####################################################################################