Light Flash software for Windows

Light installation manual

Contents:

0. Overview

1. System requirements

2. Software installation

3. Hardware installation

4. Using the software

4.1 Advanced flashing options

4.2 General options

4.3 Program

4.4 Write authority ID

5. Troubleshooting

6. Known problems

0. Overview

Light Flash software is designed to be “light”, when comparing to regular flashing system, the 	number of boxes and cables has been reduced.

1. System requirements

Light Flash system requires a PC with one free parallel port and one free serial port.

About 400 kilobytes of hard disk space is needed for program and it’s dll-files. Flash image files

needs also free space on hard disk.

Light Flash software can be used in 386-based computer but fast 486 or pentium is recommended 	because 	the speed of  download depends on the speed of the computer.

Light Flash software requires Microsoft Windows to work. Supported versions are 3.1x and 95 ver 	A and earlier. Other versions will be supported in later releases.

2. Software installation

Installation of Light Flash software is easy: just run lightpac.exe and select directory where Light application should be installed.

You can access the help file directly by double-clicking it or from Light Flash software by

selecting Help. Light Flash software does not start if there is no dongle connected and it’s

drivers installed.

Light application creates file named light.ini in windows-directory when executed for the first time.

3. Hardware installation

Hardware installation is shown in picture (This picture is included in light.hlp help file).

Components are:

Service cable		SCH-5		0730098

Flash loading adapter	FLA-5		0770085

Light Flash cable		FLC-5		0770107

Modular cable		XCM-1		4626131

Power source		ACH-6U	0270382

Security box		TDF-4		0770106

Service battery		BBD-3		0775071

DC cable		PCC-1		0770050

Serial cable		AXS-4		0730090

Dongle, no PKD-1D

Possible errors when connecting hardware:

There is a switch inside the TDF-4. It must be in front position when using Light Flash.

The other position is for regular flash system.

Be sure you connect the modular cable XCM-1 from TDF-4 to FLC-5. It can be connected from

FLA-5 to FLC-5 but then the system will not work.

FLA-5 and TDF-4 should be powered off when connecting cables.

4 Using the software

After you have succesfully installed both the software and the hardware, you can download

programs to phones and write authority ID’s.

(Pictures are taken with windows 95)

Start program by double-clicking the light.exe-icon. (remember: If you don’t have dongle and

it’s driver installed, software will not start).

When running program for the first time, you need to select some options in general options-

dialog.

(Main window of Light Flash application)

4.1 Options / Advanced flashing options

Read configuration from file:

Usage:

If the phone doesn’t start when pressing the power-button or if you accidentally

downloaded the wrong software to phone, select this before downloading.

Explanation:

Light Flash software reads phone’s hardware information from flash image file

instead of asking it from phone.

4.2 Options / General options

Write authority ID automatically:

If you want Light Flash software to write authority ID automatically after

downloading, select this option.

Service battery connected:

If you have service battery connected to your system, select this.

If you are using regular battery, unselect this.

No automatic identification:

If you don’t want phone to be identified automatically, select this.

Automatic identification requires TDF-4.

TDF-4 in port... :

Select this if you have TDF-4 connected in your Light Flash system.

Port is the serial port where TDF-4 is connected.

4.3 Execute / Program

Select this when you want to download flash image file to phone.

Depending on your choices in general options, software will try to identify the phone.

If you are not using service battery, software will prompt you to switch phone on and off

when needed.

When the phone is identified, software asks you the name of the file(s) to be downloaded.

After selecting file(s), follow the instructions gived by the program.

Note that phone can be powered on even if there is nothing on the display of the				phone (minimum mode).

4.4 Write authority ID

If you want to write authority ID to phone, select this.

You may want to write authority ID if the automatic ID writing fails after downloading.

5 Troubleshooting

In case of any trouble first make sure your hardware connections are ok.

Problem:					Try this:

Any prolem					Check connections, is power source type 								right (ACH-6)?

Download ok, phone does not start			Wrong type flash image file?

Authority ID writing failure			Is phone switched on?

(see "known problems")				Select Execute/Write authority ID

Wrong file downloaded.				Download the right program, but first

Phone does not start.				select "Use configuration from file" in

"Options/Advanced flashing options".

"Ext ram failure"-error				Flash file may be corrupted or phone’s

external ram has errors.

"Serial data failure"-error				Are you flashing NSB-3 with service 								battery? (see "known problems")

Software causes computer lock-up			Remove light.ini-file located in windows-

Directory.  Start light.exe and select general

Options.

Other things to try:

Switch TDF-4 and FLA-5 off, close program, remove battery and cables from phone, connect again, start program and try again.

6. Known problems

HD953 (or NSB-3) does not work with service battery in some computers. Use regular battery. (Remember to unselect “Service battery connected” in general options).

Automatic authority ID writing fails sometimes because phone does not start always as quickly

as expected .

Computers with power saving features may have problems if power save mode is activated during

Downloading.

Problems has been discovered when using Light Flash in Compaq, ICL and HP desktop 	computers. Sometimes Light will not work with some PC and after changing the PC

to identical PC (same manufacturer, same model) it works.