ALPINE TEST BIOS RELEASE NOTE

VERSION DATE: 07/26/93

1. This BIOS is the same as 542x BIOS vervion 1.30 except this BIOS

saves/restores SR1F register before/after any SRF write operation.

VERSION DATE: 07/28/93

1. Set bit 7 of SR16 to be 0 at the beginning of the VGA BIOS POST routine

in order to solve VL Bus problem.

2. Add new video mode 74h @60 Hz.

VERSION DATE: 08/05/93

1. Add new modes:

(a) Mode 72h @56Hz, 800*600*32bit (16M colors).

(b) Mode 6Dh @60Hz, 1280*1024*8bit (110 Mhz MCLK).

(c) Mode 73h @86iHz, 1024*768*32bit (16M colors).

2. Temporarily delete Mode 6Dh @86iHz due to the BIOS support issue.

3. Support 1,2,4 MB video memory size detection. No more 256k/512k support.

VERSION DATE: 08/18/93

1. Support new 1280*1024 monitor type in BIOS, the scratch register SR9 has

new definitions in bit 1:0 which are 00 = 1280*1024*87i monitor type

01 = 1280*1024*60Hz monitor type

10 = 1280*1024*70Hz monitor type

[NOTE]: The current CLMODE doesn't support this feature, user should directly

change bit 1:0 of SR9 in order to get 60Hz monitor type mode.

2. Completely support all required modes for ALPINE as follows:

a) mode 14h@70Hz, text mode

b) mode 54h@70Hz, text mode

c) mode 55h@70Hz, text mode

d) mode 6Ah/58h@56Hz, 800*600*16

mode 6Ah/58h@60Hz, 800*600*16

mode 6Ah/58h@72Hz, 800*600*16

e) mode 5Ch@56Hz, 800*600*256

mode 5Ch@60Hz, 800*600*256

mode 5Ch@72Hz, 800*600*256

f) mode 5Dh@87i,  1024*768*16

mode 5Dh@60Hz, 1024*768*16

mode 5Dh@70Hz, 1024*768*16

mode 5Dh@72Hz, 1024*768*16

g) mode 5Fh@60Hz, 640*480*256

mode 5Fh@72Hz, 640*480*256

h) mode 60h@87i,  1024*768*256

mode 60h@60Hz, 1024*768*256

mode 60h@70Hz, 1024*768*256

mode 60h@72Hz, 1024*768*256

i) mode 64h@60Hz, 640*480*64K

mode 64h@72Hz, 640*480*64K

j) mode 65h@56Hz, 800*600*64K

mode 65h@60Hz, 800*600*64K

mode 65h@72Hz, 800*600*64K (new)

k) mode 66h@60Hz, 640*480*32K

mode 66h@72Hz, 640*480*32K

l) mode 67h@56Hz, 800*600*32K

m) mode 6Ch@87i,  1280*1024*16

mode 6Ch@60Hz, 1280*1024*16 (110Mhz VCLK) (new)

n) mode 6Dh@87i,  1280*1024*256

mode 6Dh@60Hz, 1280*1024*256 (110Mhz VCLK) (new)

o) mode 70h@70Hz, 320*200*16M (test purpose, will be deleted eventually)

p) mode 71h@60Hz, 640*480*16M

q) mode 72h@56Hz, 800*600*16M (new)

mode 72h@60Hz, 800*600*16M (new)

r) mode 73h@87i,  1024*768*16M (new)

s) mode 74h@87i,  1024*768*64K

mode 74h@60Hz, 1024*768*64K (new)

mode 74h@70Hz, 1024*768*64K (new)

mode 74h@72Hz, 1024*768*64K (new)

t) mode 75h@87i,  1280*1024*64K (new)

u) mode 76h@60Hz, 640*480*16M (new)

mode 76h@72Hz, 640*480*16M (new)

VERSION DATE: 08/30/93

1. Put SR0F (bit6:5 only) value back to each extended mode table which of item

was accidentally deleted from the last release version.

VERSION DATE: 09/28/93

1. Change mode 6Dh@60Hz, 1280*1024*256 (110Mhz) by using VCLK/2, hidden DAC

register = 4Ah.

2. Not to set SRF bit6=1 during Font loading except Text mode

(mode 14h,54h,55h).

3. One source code can create two different kind of binary code,

(a) VL/ISA VGA BIOS: type "BIOS.bat", produce "VGA.BIN" & "VGA.EXE".

(b) PCI VGA BIOS: type "BIOS.bat P", produce "VGA_PCI.BIN" & "VGA_PCI.EXE".

4. * WARNING * No space for Motherboard version BIOS.

-----------------------------------------------------------------------------

* Version 1.00 �01   Date: 9/30/93

-----------------------------------------------------------------------------

1. Formal release version of ALPINE BIOS.

2. Change SR16 to be default value 22h for all modes.

3. * WARNING * No space for Motherboard version BIOS.

-----------------------------------------------------------------------------

* Version 0.02 �01   Date: 10/27/93

-----------------------------------------------------------------------------

1. Change SR16 to be default value 31h for all modes.

2. Take out 8x14 font, replaced with 8x16 font in the VGA BIOS.

-----------------------------------------------------------------------------

* Version 0.03 �01   Date: 11/04/93

-----------------------------------------------------------------------------

1. Add refresh 71Hz to 1280x1024x256.  (CLVESA.ASM)

2. Add VESA Display Power Management Signaling.  (ANA_EXT.INC, EXT.ASM)

-----------------------------------------------------------------------------

* Version 0.04 �01   Date: 11/09/93

-----------------------------------------------------------------------------

1. Modify SR16 to set b6 for SIS460 chip set.  (REGTBL.ASM)

2. Remove patch for 1280x1024x256 @71Hz.  (EXT.ASM)

-----------------------------------------------------------------------------

* Version 0.05 �01   Date: 11/29/93

-----------------------------------------------------------------------------

1. Add 1024x768x32K(Vesa #116/Mode #79) and 1280x1024x32K(Vesa #119/Mode #78).

(ANA_EXT.INC)

2. Fix memory requirement for modes 73 and 75.  (ANA_EXT.INC)

3. Replace refresh rate 72Hz by 75Hz.  (ANA_EXT.INC, EXT.ASM)

4. Add new function call AH=12h, BL=9Eh, BH=XXh, AL=02h to set Memory Clock.

(XALTSEL.ASM)

5. Load TSR 8X14 Font.  (VGAB.ASM, VGAG.ASM, VGAUTIL.ASM, VGAE.ASM)

6. Fix Save/Restore Video State, (AH=1Ch, AL=01h/02h)  (RW_STATE.ASM)

7. Fix Inquire VGA Type (AH=12h, BL=80h)  (XALTSEL.ASM)

8. Memory detection.  (AVGA.ASM)

-----------------------------------------------------------------------------

* Version 0.06 �01   Date: 12/03/93

-----------------------------------------------------------------------------

1. Fix SuperVGA(800x600x16) @75Hz issue.  (EXT.ASM)

2. Modify DPMS for new mode "Standby".  (CLVESA.ASM)

-----------------------------------------------------------------------------

* Version 0.07 �01   Date: 12/10/93

-----------------------------------------------------------------------------

1. Fix MicroSoft Hebrew's bug.  (VGAG.ASM)

2. Re-arrange PCI header for OEMSI.  (VCP.ASM, VGAPOD.ASM)

-----------------------------------------------------------------------------

* Version 0.07 �01   Date: 12/21/93

-----------------------------------------------------------------------------

1. Fix DisplayMate (DMU.EXE) bug.  (VGAG.ASM)

-----------------------------------------------------------------------------

* Version 0.07 �02   Date: 12/23/93

-----------------------------------------------------------------------------

1. Fix 8x14 font TSR problem.  (VGAB.ASM)

-----------------------------------------------------------------------------

* Version 0.08 �01   Date: 01/13/94

-----------------------------------------------------------------------------

1. This BIOS v0.08 �1 complies with New Spec.  (STRUCT.INC, ANA_EXT.INC, EXT.ASM)

2. The display disappeared after extended function A1 call (Read monitor ID).

The problem was video disabled. It is fixed in this version at the module

"Determine_analog_monitor".  (VGAPOD.ASM)

3. The 75Hz resolution timing for all modes have been re-adjusted to match

VESA specification(VDMT 75HZ Rev. 1.2P, Proposed on June 16. 1993)

(EXT.ASM)

4. The ext-function call 9E (SetMemClK) has been modified to save b7 & b6 of

SR1F and cleared b7 & b6 of BH.  (XALTSEL.ASM)

5. The mode 5Eh 640x400x256 @70Hz is added.  (ANA_EXT.INC)

6. Removed the mode 70h 320x200x16M.  (ANA_EXT.INC)

7. Function 9Ah "Inquire User Options" was modified to comply with New Spec.

(XALTSEL.ASM)

8. Function A4h "Set Monitor Type (Vertical)" will set Horizontal Monitor Type

to "0".  (XALTSEL.ASM)

9. The refresh rate 72Hz was put back.  (ANA_EXT.INC, EXT.ASM)

10. Set b1 to "0" of GR06 for 32K/64K modes.  (EXT.ASM)

11. Take the top 14 bytes of 8x16 fonts as 8x14 fonts.  (VGAE.ASM, VGAUTIL.ASM)

-----------------------------------------------------------------------------

* Version 0.08 �02   Date: 01/24/94

-----------------------------------------------------------------------------

1. Fix DPMS issue for the Standby mode and return correct status.  (CLVESA.ASM)

2. Modify CR04 from 53h to 54h for the mode 76h @60Hz.  (EXT.ASM)

3. Add 3 mode refresh rates (60/72/75Hz) to the mode 67h.  (ANA_EXT.INC)

4. Adjust the CRTC timing for 75Hz refresh rate.  (EXT.ASM)

5. Fix Save/Restore issue.  (RW_STATE.ASM)

6. Remove 72Hz from the modes 5Dh, 60Hz, 68Hz, and 74Hz.

7. Fix 640x480@75Hz.  (EXT.ASM, ANA_EXT.INC)

8. Fix Fun 9Eh (Set Memory Clock) problem.  (XALTSEL.ASM)

9. Modify PCI Device Id from A4h to A8h.  (VCP.ASM)

-----------------------------------------------------------------------------

* Version 0.08 �03   Date: 02/02/94

-----------------------------------------------------------------------------

1. Fix resolution 800x600x32bit (vesa 115h) issue in VPIC.  (CLVESA.ASM)

2. Fix 640x480x32k(mode 66) and 640x480x64k(mode 64) @60Hz.  (EXT.ASM)

3. Modify for loading TSR 8x14 font in agreed way.  (VGAB.ASM)

4. Fix rebooting problem for MonType.  (XALTSEL.ASM)

5. Set the requested value for detecting Color/Mono.  (VGAPOD.ASM, OEM.ASM)

6. Fix Read Character/Attribute (AH=08h) issue.  (VGAE.ASM)

7. Fix for Fun 1Ch w/o request.  (RW_STATE.ASM)

-----------------------------------------------------------------------------

* Version 0.08 �04   Date: 02/09/94

-----------------------------------------------------------------------------

1. Modify FIFO threshold of SR16 from 01h to 0Ah for modes 73h, 74h, 75h, 68h,

and 69h.  (ANA_EXT.INC)

2. Remove refresh rate 60Hz from the mode 6Ch (1280x1024x16).  (ANA_EXT.INC)

3. Don't modify DX value for calling Vesa Function 05h -- Select Super VGA

Memory Window.  (CLVESA.ASM)

4. Don't program SR1F for MCLK in DPMS.  (CLVESA.ASM)

5. Initialize b0 of SR17 to '1' during Power_On_Diagnostics only.

(VGAPOD.ASM, REGTBL.ASM)

-----------------------------------------------------------------------------

* Version 1.00   Date: 02/18/94

-----------------------------------------------------------------------------

1. Modify memory required in supplemental table.  (ANA_EXT.INC)

2. Fix CSHOW's problem at 1280x1024x16.  (CLVESA.ASM)

3. Add 5430 ID for Inquire VGA Type.  (XALTSEL.ASM)

4. Modify Vesa function (AX=4F06h) for 16 colors.  (CLVESA.ASM)

Note: All modified spots are marked by "v0.08B5".

-----------------------------------------------------------------------------

* Version 1.00a   Date: 03/01/94

-----------------------------------------------------------------------------

1. Comment out clock settings for DPMS.  (CLVESA.ASM)

2. Set b0 of SR17 to '1'.  (VGAPOD.ASM, REGTBL.ASM)

3. Add extended bit b4 of CR1B to CR13 in Vesa function.  (CLVESA.ASM)

4. Fix PCI's Class Code.  (VCP.ASM)

Note: Both 1.00b and 1.00c are for INTEL PCI testing.

-----------------------------------------------------------------------------

* Version 1.00d   Date: 05/02/94

-----------------------------------------------------------------------------

1. Fix 8x14 user-defined font loading.  (VGAUTIL.ASM)

-----------------------------------------------------------------------------

* Version 1.00e   Date: 05/18/94

-----------------------------------------------------------------------------

1. Don't modify b2 & b3 of Miscellaneous Output Register in DPMS.  (CLVESA.ASM)

-----------------------------------------------------------------------------

* Version 1.10 �01   Date: 04/27/94

-----------------------------------------------------------------------------

Note:  This BIOS can work with 5434/5430.  It hasn't been tested by QA yet!!!

1. Save SR12 first and set b1 to access DAC extended entries; restore SR12

when done in DPMS.  (CLVESA.ASM)

2. Reset b1 & b2 of GR0E in Power_On_Diagnostics.  (VGAPOD.ASM)

3. Use relative location to the 1st byte of buffer; 3 pointers located at 10h,

12h, and 14h.  (RW_STATE.ASM)

4. Remove Fun. 9Eh, Set Memory Clock.  (XALTSEL.ASM)

5. Add 5430 detect code.  (CLVESA.ASM)

6. Support 5430 VBE/PM (DPMS).  (CLVESA.ASM)

7. Remove 8X14 font.

8. Mode 5Dh (1024x768x16@75Hz),

Mode 60h (1024x768x256@75Hz),

Mode 65h (800x600x64K@72Hz), and

Mode 67h (800x600x32K@72Hz)

are added to 5430.  (ANA_EXT.INC)

9. Extended bit to 1024x768 refresh is defined in b7 of SR0A.

10. For 5430 SR16[3:0]=1000b if SR1F[5:0] < 20h

SR16[3:0]=0110b if SR1F[5:0] >= 20h.  (VGAB.ASM)

11. For 5430 set GR18 to 03h during POST & never change after that.

(VGAPOD.ASM)

12. For 5430 set MCLK = 60.852Mhz, and SR14[2:0] = 00b during POST.  (VGAPOD.ASM)

13. 512K, 1M, and 2M video memory detection and save the result in SR0A during

POST.  (AVGA.ASM)

14. Don't modify DX value for calling Vesa Function 05h

--- Select Super VGA Video Memory Window.  (CLVESA.ASM)

15. Return correct status for DPMS Set Display Power State.  (VGAMODE.INC)

16. Modify structure of video mode table for 5430.  (STRUC.INC, ANA_EXT.INC)

-----------------------------------------------------------------------------

* Version 1.10 �02   Date: 05/12/94

-----------------------------------------------------------------------------

1. Fix extended modes setting issue.  (CLUTIL.ASM)

2. Fix user-defined 8X14 font loading.  (VGAUTIL.ASM)

-----------------------------------------------------------------------------

* Version 1.10 �03   Date: 05/13/94

-----------------------------------------------------------------------------

1. Add new functins AEh(GetHighRefresh) and AFh(SetHighrefresh) for 640X480

@72Hz/75Hz.  (XALTSEL.ASM)

2. Modify CRTC timing for 640X480@72Hz/75Hz, 800X600@60Hz/71.28Hz/75Hz, and

1280X1024@60Hz/71.18Hz/75Hz.  (ANA_EXT.ASM, EXT.ASM)

3. Include SR17 in Save/Restore Fun. 1Ch.  (RW_STATE.ASM)

4. CR1C needs to be reset at begining of Video_Set_Mode.  (VGAB.ASM)

5. Just program MCLK(SR1F) once.  (VGAPOD.ASM)

-----------------------------------------------------------------------------

* Version 1.10 �01   Date: 05/25/94

-----------------------------------------------------------------------------

1. Modify 5430 memory detection for 1/2M, 1M, or 2M.  (AVGA.ASM)

2. NCR's monitors need waiting while video mode setting(VCLK changes).

(VGAB.ASM)

3. Place 640X480@75Hz as default in SR14[4:3].  (VGAPOD.ASM)

4. Keep MemClk = 50Mhz for 5430.  (REGTBL.ASM, OEM.ASM)

-----------------------------------------------------------------------------

* Version 1.10 �03   Date: 05/26/94

-----------------------------------------------------------------------------

Note: Version 1.10 �02, Dated 05/25/94, did not go thru QA.

1. Reset PCI EROM enable bit during Power_On_Diagnostics.  (VGAPOD.ASM)

-----------------------------------------------------------------------------

* Version 1.10 �04   Date: 05/31/94

-----------------------------------------------------------------------------

1. Fix CRTC timing for 640X480@72.85Hz/75Hz.  (EXT.ASM)

-----------------------------------------------------------------------------

* Version 1.10 �05   Date: 06/07/94

-----------------------------------------------------------------------------

1. Let 640X480@72.85Hz or 75Hz be OEMSIable.  (VGAPOD.ASM, REGTBL.ASM)

2. Modify 800X600X32K/64K@60Hz settings.  (EXT.ASM, ANA_EXT.INC)

3. Modify video memory detection for IBM PS/1 warm boot issue.  (AVGA.ASM)

4. Set CR1B[5] = 1 for mode 5Fh.  (ANA_EXT.INC)

-----------------------------------------------------------------------------

* Version 1.10A  Date: 06/15/94

-----------------------------------------------------------------------------

1. Modify 640X480X32K/64K@60Hz, @72Hz, or @75Hz settings running with only 1MB

memory.  (ANA_EXT.INC EXT.ASM)

-----------------------------------------------------------------------------

* Version 1.10B  Date: 06/28/94

-----------------------------------------------------------------------------

1. Let video modes be set if MCLK is greater than 57MHz.  (ANA_EXT.INC)

2. Reset b0 of SR07 while doing 1MB memory detection.  (AVGA.ASM)