# Setting Up TMS320 DSP Interrupts IN C

Leor Brenman

Digital Signal Processor Products - Semiconductor Group Texas Instruments

Table of Contents

µ1.	Introduction	1

2.	Creating an Interrupt Service Routine	2

2.1.	Naming Convention	2

2.2.	Calling Other Functions	2

2.3.	ISR Contents	2

3.	Setting up Interrupt Vectors	3

3.1.	Vector Table Locations	3

3.2.	Using a Named ASM Section	4

3.2.1.	Handling Reserved and Unused Locations	5

3.2.2.	Linking Into Memory Map	6

3.3.	Boot Loader	6

3.4.	Run-time Vector Installation	7

3.5.	Vector Table Pointers	7

3.5.1.	TMS320C4x Example Using a C In-Line ASM statement	7

3.5.2.	TMS320C4x Example Using the PRTS	7

3.5.3.	TMS320C5x Example using Memory-Mapped Registers	8

3.5.4.	A More Flexible Method	8

4.	Enabling Interrupts	10

4.1.	Interrupt Flag Registers	10

4.2.	Interrupts Enable Registers	10

4.3.	Global Interrupt Enable Bit	11

4.4.	Initialization	11

4.4.1.	Assembly Language	11

4.4.2.	Using C Pointers to Memory Mapped Registers	12

5.	Enabling Interrupt Sources	13

6.	Summary	13

A.	Related Documents	14

B.	Interrupt Vector Maps	15

B.1.	TMS320C2x Interrupt Vector Map	15

B.2.	TMS320C26 (Microcomputer/Bootloader mode) Interrupt Vector Map	16

B.3.	TMS320C3x Interrupt Vector Map	17

B.4.	TMS320C31 (Microcomputer/Bootloader mode) Interrupt Vector Map	18

B.5.	TMS320C4x Interrupt Vector Map	19

B.6.	TMS320C4x Trap Vector Map	21

B.7.	TMS320C5x Interrupt Vector Map	22

C.	Code Examples	23

C.1.	TMS320C25 Complete Example	23

C.2.	TMS320C26 Complete Example	27

C.3.	TMS320C30 Complete Example	32

C.4.	TMS320C31 Complete Example	40

C.5.	TMS320C40 Complete Example with the PRTS	46

C.6.	TMS320C40 Complete Example without the PRTS	50

C.7.	TMS320C50 Complete Example	56

# Setting Up TMS320 DSP Interrupts IN C

# 1.	Introduction

One of the inherent differences between a digital signal processor (DSP) and other processors is it's ability to receive and service multiple sources of interrupts very quickly. These interrupt sources could be timers for setting up different time bases, external analog-to-digital converters for converting analog audio signals into digital data, or on-chip coprocessors indicating a task is complete. Typically, when an interrupt occurs, ordinary processing stops and an interrupt service routine (ISR) starts executing. The ISR "services" the interrupt by resetting a timer period register, reading or writing data to a peripheral, or setting up a new task, for example. Interrupt sources on the TMS320 family of DSPs include:

1) Reset

2) External interrupt pins

3) On-chip Peripherals

4) On-chip DMA (Direct Memory Access)

5) Traps (Software Interrupts)

DSP user's guides define exactly how a DSP reacts to an interrupt. That is, (1) the interrupt latency - the time it takes from the interrupt occurring (a low pulse on an external interrupt pin) to the execution of the first instruction in the ISR, (2) the interrupt priorities, and (3) conditions during the ISR, such as the ability to accept another interrupt. The user's guide also explains how to set up a DSP to handle interrupts. That is, what registers need to be initialized and what memory location to initialize with the address of the ISR. This is usually done in the native assembly language or in a high-level language.

Programming DSPs in a high level language such as C provides for portability and maintainability. A program can be rapidly prototyped and proven in C and then optimized to a particular processor architecture. Often, the real-time, or time critical, portions of the code are hand assembled in this optimization process resulting in high performance, yet, efficient and readable code.

Texas Instruments offers Optimizing ANSI C Compilers for the TMS320C2x, TMS320C3x, TMS320C4x and TMS320C5x DSPs. These compilers produce efficient code for these high performance processors. While these DSPs are different, setting up interrupts for them is fairly similar. The main steps for setting up interrupts for TMS320 DSPs are as follows:

1) Create the interrupt service routines

2) Set up the vectors in the memory map

3) Enable the interrupts to the CPU

4) Enable the interrupt sources

This document describes different methods of setting up interrupts for these processors in C. When C cannot be used C callable assembly language routines or in line C statements are used. Topics covered in this document include:

1) C In-line assembly language

2) C-callable assembly language modules

3) Assigning symbols at link time

4) C40 Parallel Runtime Support Library (PRTS)

5) Initializing peripherals in C

6) Individual placement of C variables/arrays in DSP memory map

7) Installing interrupt vectors at run-time

8) Accessing memory mapped CPU registers in C

While examples are contained throughout this document, Appendix C contains complete examples of setting interrupt vectors for all the processors described herein.

# 2.	Creating an Interrupt Service Routine

The first step in setting up interrupts in C is to define the ISR. The ISR is no different than an ordinary C subroutine except for the name of the routine.

## 2.1.	Naming Convention

By naming the subroutine c_intnn, where nn can be 00 to 99, the floating-point DSP C compiler can identify the routine as an ISR and can follow different rules for saving and restoring registers. Ordinary routines save and restore registers that are used by the routine according to the rules defined in the appropriate C compiler manual. Interrupt service routines need to save and restore every register that is used by the ISR. The following example shows an ISR defined in C:

void c_int11(void)

{

receiveCounter++;

}

For readability, it is possible to use a macro definition to rename c_intnn to a more descriptive name. For example:

#define serialPortReceiveISR	c_int11(void)

void serialPortReceiveISR(void)

{

receiveCounter++;

}

For the fixed-point DSP C compiler the ISR name must be c_intn, where n can be 0 to 9. Also, in the fixed-point DSP compiler all registers are saved and restored, using the runtime support library functions I$$SAVE and I$$REST.

There is no correlation between the ISR name and it's interrupt functionality. However, c_int00 (for floating-point DSPs) and c_int0 (for fixed-point DSPs), are defined in the respective runtime support library as the reset routines for initializing the C environment. These routines are found in the rts.src archive library, in the file boot.asm. Note that C variables and function names are accessed in assembly language using the underscore symbol (_) preceding the label. For example, the function c_int00() could be executed in assembly language by branching to label _c_int00.

Note that no parameters can be passed to an ISR and nothing can be returned from an ISR.

## 2.2.	Calling Other Functions

In the floating-point DSP C compiler, when an ISR calls another function, the C compiler forces the ISR to save and restore all CPU registers, not just the ones used in the ISR. This occurs since the compiler has no guaranteed way of knowing what registers are used in the called function (the function may be externally defined). Therefore, in order to minimize the length of the ISR avoid calling other functions from within the ISR.

## 2.3.	ISR Contents

What the ISR does depends on the type of interrupt processing that is required. In general, since no parameters are passed to, or from, an ISR, global variables are required. In the previous example, the variable receiveCounter would have been declared as a global variable. If the ISR is servicing an interrupt from an on-chip peripheral, then the peripheral registers would be read from and/or written to. Following is an example of an ISR that services the on-chip serial port that has an A/D and D/A connected to it. Data is transferred to and from the serial port registers and C variables, input and output. All of the variable in this ISR have been declared globally.

void c_int05(void)

{

/* Get input value read in from A/D connected to serial port */

input = *serPortRec;

/* Write output value to D/A connected to serial port */

*serPortTrans = output;

}

# 3.	Setting up Interrupt Vectors

After creating the ISR, the vector (or address) of the ISR must be appropriately loaded into memory. In general, these vectors are located at address location 0x0 in (program) memory. However, some of the processors require, or provide for, installing the interrupt vectors in other locations. Appendix B contains interrupt vector tables that illustrate the location in (program) memory that the interrupt vectors should be installed for each of the TMS320 DSPs.

As an example, a portion of the TMS320C3x interrupt vector table is shown below:

Interrupt Vector Locations for the TMS320C3x

| Interrupt | Memory Location | Function |
| --- | --- | --- |
| RESET | 0x0 | External Reset |
| INT0_ | 0x1 | External Interrupt 0 |
| INT1_ | 0x2 | External Interrupt 1 |
| INT2_ | 0x3 | External Interrupt 2 |
| INT3_ | 0x4 | External Interrupt 3 |
| XINT0 | 0x5 | Serial Port 0 Transmit |
| RINT0 | 0x6 | Serial Port 0 Receive |
| . | . | . |
| . | . | . |
| . | . | . |

## 3.1.	Vector Table Locations

For the TMS320C25, TMS320C28, TMS320C26 in Microprocessor Mode, TMS320C30 and the TMS320C31 in Microprocessor Mode, the vectors always start at location 0x0. For the TMS320C31 in Microcomputer/Bootloader mode, the vectors start at location 0x809fc1. For the TMS320C26 in Microcomputer/Bootloader mode, the vectors start at location 0xffa0. For the TMS320C5x , reset is always at location 0x0, but the interrupt vectors can reside on any 2k word page in program memory. The vector table location is related to the value of the IPTR bits of the PMST register. Valid interrupt vector table base addresses for the TMS320C5x are 0x0, 0x800, 0x1000, 0x1800, 0x2000, ..., 0xf800.

For the TMS320C4x, reset can be located at one of four locations as defined by the external pins RESETLOC0 and RESETLOC1. The TMS320C4x's interrupt vectors can reside on any 512 word boundary in memory. The vector table location is defined by the value of the word stored in the IVTP (Interrupt Vector Table Pointer) register. Additionally, The TMS320C4x's Trap interrupt vectors can reside on any 512 word boundary in memory. The Trap vector table location is defined by the value of the word stored in the TVTP (Trap Vector Table Pointer) register. Valid interrupt vector, or trap, table base addresses for the TMS320C4x are 0x0, 0x200, 0x400, 0x600, 0x800, 0xa00, 0xc00, 0xe00, 0x1000, 0x1200, ..., 0xfffffe00.

This information is summarized in the table below. For further explanation and detail, refer to the appropriate device user's guide.

Interrupt Vector Table Locations for the TMS320 DSPs

| Processor | Vector Table Base Address | Comments |
| --- | --- | --- |
| TMS320C2x | 0x0 | Not including TMS320C26 in Microcomputer/Bootloader Mode |
| TMS320C26 | 0xfa00 | Microcomputer/Bootloader Mode |
| TMS320C30 | 0x0 |  |
| TMS320C31 | 0x0 | Microprocessor Mode |
| TMS320C31 | 0x809fc1 | Microcomputer/Bootloader Mode |
| TMS320C4x RESET | 0x0, 0x7fff ffff, 0x8000 0000, 0xfff ffff | Reset vector location defined by external pins RESETLOC0 and RESETLOC1 |
| TMS320C4x Interrupt Vectors | Any 512 word boundary | Interrupt vector table base address defined by value in IVTP register |
| TMS320C4x Trap Vectors | Any 512 word boundary | Trap vector table base address defined by value in TVTP register |
| TMS320C5x RESET | 0x0 |  |
| TMS320C5x Interrupt Vectors | Any 2K word page | Related to value of IPTR bits of the PMST register |

## 3.2.	Using a Named ASM Section

There are two methods of placing the interrupt vectors in the memory map at the appropriate location. The most straightforward method is to create a table in assembly language using the named section assembler directive .sect. This table contains the addresses (or branch-to-addresses) of the interrupt vectors.

The TMS320C2x, the TMS320C5x and the TMS320C31 in Microcomputer/Bootloader Mode execute the code at the interrupt vector locations. Therefore "branch-to-address" instructions must be used as interrupt vectors. For the TMS320C31 in Microcomputer/Bootloader mode use the 24-bit branch instruction BR to enable branching to any location in the address space. For example, on the TMS320C31, interrupt vector 1 could be defined in assembly language as follows:

INT1:	br	_c_int01

On the TMS320C30, the TMS320C31 in Microprocessor Mode, and the TMS320C4x the value at the interrupt vector is used as the address of the next instruction to be fetched. Therefore, the address of the appropriate ISR must be stored at the interrupt vector location using the .word assembler directive. For example, on the TMS320C4x, interrupt vector 1 could be defined in assembly language as follows:

INT1:	.word	_c_int01

Note the underscore symbol (_) preceding the c_int01 function name in the previous two examples.

Since the ISR labels are declared external to the assembly language module, the labels must be declared as .ref or .global. Following is an example of an assembly language module, vecs.asm, that defines a named section containing TMS320C5x branch-to-address vectors:

.ref	_cint0, _c_int9		;reference interrupt vectors defined externally

.sect	"vectors"		;declare a named section "vectors"

RS:	b	_c_int0			;branch to reset vector

I1:	b	_c_int9			;branch to interrupt vector 1

I2:	b	_c_int9			;branch to interrupt vector 2

### 3.2.1.	Handling Reserved and Unused Locations

Sometimes the interrupt vector table contains reserved locations, as in the case of the TMS320C26 in Microcomputer/Bootloader mode or when the TMS320C4x or TMS320C5x reset and interrupt vectors are not contiguous. This also occurs on spinoff devices with missing peripherals such as the TMS320C31. Also, your system may not use all of the interrupts available. In order to handle these "holes" in the vector map, the .space assembler directive may be used. Note that on the fixed point devices, .space reserves bits, while on the floating point devices .space reserves words. For example, on the TMS320C26 in Microcomputer/Bootloader if you where to use every interrupt available your vector map may look like this:

.sect		"vectors"		;define named section

;for reset & interrupt vectors

.space		2*16			;reserved

b		_c_int1			;INT0

b		_c_int2			;INT1

b		_c_int3			;INT2

b		_c_int4			;TINT

b		_c_int5			;RINT

b		_c_int6			;XINT

b		_c_int7			;TRAP

Note that the space directive reserves one location for the reset vector that is not employed in bootloader mode since reset invokes the bootloader in Microcomputer/Bootloader mode. Here, the section, "vectors" would be linked to location 0xfa00. Alternatively, the .space directive could have been removed and the section linked to location 0xfa02.

If, however, only the timer and trap interrupt vectors are to be used, the vector map could be defined as follows using the .space directive:

.sect		"vectors"		;define named section

;for reset & interrupt vectors

.space		2*4*16			;reserved and 3 unused vectors

b		_c_int4			;TINT

.space		2*2*16			;2 unused vectors

b		_c_int7			;TRAP

### 3.2.2.	Linking Into Memory Map

Once the named section is created, the name of the section ("vectors" in the last example) can be used as a "handle" to link the table to the appropriate location in the memory map, using the TMS320 linker, LNK30.EXE or DSPLNK.EXE, for floating-point or fixed-point DSPs, respectively. There are three steps to doing this:

1) Link in the assembly language module.

2) Define a linker MEMORY section corresponding to the interrupt vector locations.

3) Use the linker SECTIONS area to "place" the named section into the previously defined MEMORY section.

Continuing the previous example, the following is a sample linker command file segment for the TMS320C5x:

-c

.

vecs.obj						/* (1) Link in the vector table */

main.obj

.

-lrts50.lib

MEMORY

{

PAGE0:VECTORS:	origin = 0000h, length = 0002fh	/* (2) Declare mem for vectors */

ROM:		origin = 0030h, length = 007cfh

.

}

SECTIONS

{

"vectors":	{} > VECTORS			/* (3)  Place vector table */

.text:		{} > ROM

.

}

## 3.3.	Boot Loader

When using the boot loader on the TMS320 DSPs, the default entry point for code execution, is the destination address of the first word transferred by the on-chip boot loader. The Hex conversion boot loader utilities (HEX30.EXE and DSPHEX.EXE) have provisions for overriding this and defining the entry point to be the reset routine (e.g. c_int0 or c_int00). When using the device in bootloader mode, there is no reset ISR; instead, resetting the device causes the boot loader to execute.

## 3.4.	Run-time Vector Installation

Another method that is useful during development and debugging is to install the vectors at run-time. That is, at run-time, load the address of the ISR into the proper location using a C statement. This is only appropriate for the TMS320C30, TMS320C31 in Microprocessor Mode and the TMS320C4x since they use addresses and not branch-to-addresses as interrupt vectors. The main idea is to use a C typecast to put the address of the ISR into the desired memory location. For example, on the TMS320C30, location 0x1 corresponds to external interrupt 0 (INT0_). To install the ISR c_int99() there, use the following statement:

*((void (**) ()) 0x1) = c_int99;

Here, location 0x1 is being typecasted as a pointer to a pointer a function since it contains the address of the function c_int99(). The danger in doing this is that  the C programmer might overwrite data or program memory that is allocated by the linker.

## 3.5.	Vector Table Pointers

The TMS320C5x and TMS320C4x devices have provisions for placing interrupt vectors to locations other than 0x0. Both have registers to enable the processor to identify the location of the vectors. The TMS320C4x, in addition, can define the reset vector to reside in one of four locations as determined by pins on the processor. In order for interrupts to be processed correctly, their interrupt vector table pointers must be initialized prior to receiving any interrupts. The following four examples illustrate techniques for initilizing these registers.

### 3.5.1.	TMS320C4x Example Using a C In-Line ASM statement

Following is an example of using a C In-line assembly language instruction to set the TMS320C4x interrupt vectors to start at location 0x0 by setting the value of the IVTP register to 0x0 in a hard-coded fashion.

void main(void)

{

.

asm("\t LDI \t 0x0,IVTP");

.

}

### 3.5.2.	TMS320C4x Example Using the PRTS

Following is an example of using the TMS320C4x parallel Runtime Support Library to set the TMS320C4x interrupt vectors to start at location 0x02ff800, the start of RAM block 0, by setting the value of the IVTP register using the set_ivtp() PRTS library function. When using the PRTS, no named section of interrupt vectors is required by the user. Instead, the install_int_vector() PRTS function is used to install vectors at run-time into the predefined section ".vector". In this method vectors are installed at runtime in a fashion that ensures that no program or data will be overwritten. First, the PRTS library is linked to the program and the predefined section, ".vector", is allocated to reside at the start of RAM Block 0 using a linker command file as follows:

.

-lprts40.lib

.

MEMORY

{

RAM0:	org = 0x2ff800	len = 0x400	/* RAM Block 0 */

.

}

SECTIONS

{

".vector":	{} > RAM0	/* Allocate space for interrupt */			.				/* vectors for C40 PRTS */

.

}

The main program must include the header file intpt40.h. The set_ivtp() function can now be called using the predefined argument DEFAULT and the interrupt vectors can be installed using the install_int_vector() function as follows:

#include <intpt40.h>

void c_int99(void)

{

for(;;);

}

void main(void)

{

set_ivtp(DEFAULT);			/* Initialize the IVTP register */

install_int_vector((void *) c_int99, 2);		/* Install timer interrupt */

.

}

### 3.5.3.	TMS320C5x Example using Memory-Mapped Registers

The following example sets the TMS320C5x interrupt vectors to start at location 0x800 by setting the value of the IPTR bits of the PMST register to 0x800 in a hard-coded fashion in C using a pointer to the memory mapped register, PMST, which is located at address 0x7.

void main(void)

{

unsigned int *pmst = (unsigned int *) 0x7;	/* PMST register */

*pmst = 0x800;				/* Initialize IPTR bits of PMST */

.

}

### 3.5.4.	A More Flexible Method

While the TMS320C5x C Compiler does not have a PRTS library to assist in setting up the vector table pointer registers, there is a flexible, portable method for accomplishing the same task on both the TMS320C4x and TMS320C5x. This method employs assigning symbols at link time.

The idea is to use an assembly language section (.sect) that contains reset and interrupt vectors and use the linker to get a handle to the location of the placement of the interrupt vectors. This address is made accessible to the C program and can be loaded into the vector pointer on the TMS320C4x (IVTP) or the PMST register on the TMS320C5x.

Start out by defining the interrupt vectors in an assembly language module. Following is one example on the TMS320C5x. In order to get at the address of the interrupt vectors, a label is used to locate the base address of the interrupt vector table. In this example, IVECS is used.

.def	IVECS			;IVECS defined in this module

.ref	_c_int0, _c_int1, c_int2	;reference all interrupt

;vectors declared elsewhere

**************

* Reset vector

**************

.sect	"reset"			;define named section for

;reset vector

b	_c_int0			;reset vector

*******************

* Interrupt vectors

*******************

.sect	"vectors"		;define named section

;for interrupt vectors

IVECS	.space	1			;one reserved location

b	_c_int1			;interrupt vector 1

b	_c_int2			;interrupt vector 2

In the linker, use a linker assigned label to initialize a linker defined variable. In this case, the label IVECS is assigned to the variable . Continuing the last example:

-c

vecs.obj

.

.

-lrts50.lib

_vecTable = IVECS;	/* Set vecTable to point to vector table */

MEMORY

{

PAGE 0:	VECTORS:	origin = 00000h, length =  0002fh

ROM:		origin = 00030h, length =  007CFh

P_RAM:	origin = 00800h, length =  023FFh

.

.

}

SECTIONS

{

"reset"		> VECTORS

"vectors"	> P_RAM

.text:		> ROM

.cinit:		> ROM

.bss:		> RAMB0_D

.stack:		> INT_RAM

}

In the C program declare the pointer to unsigned integer, vecTable, to be extern as follows:

extern unsigned int *vecTable;

Now it can be loaded into the PMST register as follows:

unsigned int *pmst = (unsigned int *) 0x07;  /* PMST register */

*pmst |= (unsigned int) vecTable;

Using this method on the TMS320C5x or the TMS320C4x provides a flexible approach to loading the vector table pointer registers such that when the vector table is relocated in the linker command file, the C program does not need to be recompiled, only relinked.

# 4.	Enabling Interrupts

Before interrupts can be employed, the interrupts must be enabled. There are two places that interrupts are enabled. All the processors described in this document have both an interrupt mask register (interrupt enable register) and a global interrupt enable bit, in the processors status register. The interrupt mask register provides for individual control of each interrupt source to the CPU. The global interrupt enable bit provides a master switch to turn on and off all interrupts. This bit is usually enabled once by the programmer at the beginning of the program. During interrupt processing, this bit is toggled on and off by the interrupt processing logic and the return-from-interrupt instruction that ends the ISR. This is done to prevent an ISR from being pre-empted. The user can override this by re-enabling global interrupts in the ISR.

Each processor also has an interrupt flag register. The individual bits of these registers are automatically set when an interrupt occurs. They are automatically cleared when an interrupt is taken. It is customary before enabling interrupts for the first time to clear this register. However, on the TMS320C2x family, this register is not accessible through software. For the TMS320C3x and TMS320C4x processors the register should be loaded with 0x0 to clear all interrupts. On the TMS320C5x write a 1 to each bit to clear the interrupts.

## 4.1.	Interrupt Flag Registers

The following table identifies the interrupt flag register for each of the processors:

Interrupt Flag Registers for the TMS320 DSPs

| Processor | Interrupt Mask Register | Comments |
| --- | --- | --- |
| TMS320C2x | IFR | Interrupt Flag Register. Not accessible through software. |
| TMS320C3x | IF | Interrupt Flag Register. Not memory-mapped. |
| TMS320C4x | IIF | IIOF_ Flag Register. Not memory-mapped. |
| TMS320C5x | IFR | Interrupt Flag Register. Memory-mapped at location 0x6 |

## 4.2.	Interrupts Enable Registers

The following table identifies the interrupt mask register for each of the processors:

Interrupt Mask (Enable) Registers for the TMS320 DSPs

| Processor | Interrupt Mask Register | Comments |
| --- | --- | --- |
| TMS320C2x | IMR | Interrupt Mask Register. Memory-mapped at location 0x4. |
| TMS320C3x | IE | Interrupt Enable Register. Not memory-mapped. |
| TMS320C4x | IIE and DIE | CPU Internal Interrupt Enable Register and DMA Coprocessor Interrupt Enable Register. Not memory-mapped. |
| TMS320C5x | IMR | Interrupt Mask Register. Memory-mapped at location 0x4. |

## 4.3.	Global Interrupt Enable Bit

The following table identifies the global interrupt enable bit for each of the processors:

Global Interrupt Enable Bit for the TMS320 DSPs

| Processor | Interrupt Mask Register | Comments |
| --- | --- | --- |
| TMS320C2x | INTM | Interrupt Mask Bit in the Status Register. Accessed via EINT and DINT instructions. |
| TMS320C3x | GIE | Global Interrupt Enable Bit in the Status Register. Accessed via write to Status Register |
| TMS320C4x | GIE | Global Interrupt Enable Bit in the Status Register. Accessed via write to Status Register |
| TMS320C5x | INTM | Interrupt Mask Bit in the Status Register 0. Accessed via CLRC and SETC instructions. |

## 4.4.	Initialization

There are several methods of initializing the interrupt mask register, interrupt flag register and the global interrupt bit. These methods include using asm() statements, C callable assembly language routines and C pointers to memory mapped registers.

### 4.4.1.	Assembly Language

On the TMS320C30, TMS320C31 and TMS320C40, the interrupt registers and the global interrupt enable bit in the status register are not memory mapped and hence not accessible through C directly. Subsequently, there are two methods of accessing the registers from C. The first is to use an asm() statement in C. The other method is to create a C-callable assembly language routine that loads the registers using arguments passed from the C calling program.

The asm() statement embeds the assembly language statement directly into the C program in a hard-coded fashion. The following example illustrates the use of the asm() statement to set these registers the TMS320C3x:

void main(void)

{

asm("\t LDI \t 0h,IF");		/* Clear IF register */

asm("\t OR \t 3h,IE");		/* Enable external interrupts 0 and 1 */

asm("\t OR \t 2000h,ST");	/* Enable interrupts globally */

.

}

On the TMS320C2x and TMS320C5x processors the interrupt mask bit, INTM, of the status register controls all interrupts globally. This bit is cleared to enable interrupts using the EINT assembly language instruction on the TMS320C2x and the CLRC assembly language instruction on the TMS320C5x. The following example illustrates the use of the asm() statement to clear the INTM bit on the TMS320C5x:

void main(void)

{

asm("\t CLRC \t INTM");	/* Enable interrupts globally */

.

}

The preferred method of using C-callable assembly language routines leads to more portable code. For example, on the TMS320C40, the value for the interrupt enable register, IIE, could be passed to a C callable routine, initIIE(), defined as follows:

_initIIE:

PUSH	FP		       ;manage stack on entry

LDI	SP,FP

LDI	*-FP(2),IIE	       ;load int enable register

LDI	*-FP(1),R1	       ;manage stack on exit

BD	R1

LDI	*FP,FP

NOP

SUBI	2,SP

***	B	R1		       ;branch occurs

Following is the function prototype for the C callable assembly language routine:

extern void initIIE(unsigned int);

and an example of calling the routine is shown below:

initIIE(0x1);	/* Enable Timer 0 interrupt */

### 4.4.2.	Using C Pointers to Memory Mapped Registers

On the TMS320C2x and TMS320C5x processors the interrupt mask and flag registers are memory mapped. Hence, a C pointer can be used to access these registers directly from C. However, the INTM bit must be accessed as described in the previous section. The following example illustrates initializing the TMS320C5x interrupt registers in C, using C pointers:

void main(void)

{

unsigned int *imr = (unsigned int *) 0x4;	/* IMR register */

unsigned int *ifr = (unsigned int *) 0x6;	/* IFR register */

*imr |= 0x3;			/* Enable external interrupts 1 and 2 */

*ifr = 0x01ff;			/* Clear all interrupts */

.

}

# 5.	Enabling Interrupt Sources

Finally, after all of the interrupt vectors have been installed and the interrupts have been enabled, the interrupt sources can be enabled. For on-chip resources such as timers, serial ports, and DMA, this amounts to starting the on-chip resource as described in the device user's guide. These on-chip resources are configured and started using memory-mapped registers. The following example illustrates a routine for starting the TMS320C31 on-chip DMA to transfer data and cause an interrupt:

extern int sourceArray[];

extern int destArray[];

void dmaInit(void)

{

unsigned int *dma = (unsigned int *) 0x808000;	/* Pointer to DMA */

dma[0] = 0x0;				/* Stop DMA */

dma[4] = (unsigned int) sourceArray;		/* Load DMA Source address */

dma[6] = (unsigned int) destArray;		/* Load DMA Destination address */

dma[8] = 5;				/* Load DMA Transfer Count */

dma[0] = 0xc53;			/* Start DMA to transfer data, stop, */

/* and cause a CPU interrupt */

}

# 6.	Summary

This document describes various techniques to setup and initialize interrupts on the TMS320 DSPs in C, wherever possible. C can be used to create the interrupt service routine and initialize any memory mapped registers related to interrupts. It can also be used to install the vectors at run time, however, using a named assembly language section is the preferred method. Where memory mapped registers are not provided, it is recommended that a C callable assembly language routine that accepts register values as inputs be used to initialize those registers. Finally, the TMS320C4x C compiler provides a parallel runtime support library to handle interrupts entirely in C. An example of it's use was also discussed.

# A.	Related Documents

The following documents provide additional information on this and other DSP, or C related topics:

1) 'C2x/'C5x Optimizing C-Compiler (TI)

2) TMS320 Fixed-Point Assembly Language Tools Guide (TI)

3) TMS320 Floating-Point DSP Optimizing C-Compiler (TI)

4) TMS320 Floating-Point Assembly Language Tools Guide (TI)

5) DSP Applications with the 'C30 EVM (TI)

6) TMS320C4x Parallel Runtime Support Library (TI)

7) TMS320C3x Peripheral Control Library (TI)

8) Other App Notes Related to C and DSPs (TI)

9) The C Programming Language (K&R)

# B.	Interrupt Vector Maps

This appendix describes the interrupt vector maps for the individual TMS320 DSPs discussed in this document.

## B.1.	TMS320C2x Interrupt Vector Map

The following interrupt vector map applies to all TMS320C2x devices except for the TMS320C26 in Microcomputer/Bootloader mode.

| Interrupt Name | Memory Location | Function |
| --- | --- | --- |
| RS_ | 0x0 | External Reset |
| INT0_ | 0x2 | External Interrupt 0 |
| INT1_ | 0x4 | External Interrupt 1 |
| INT2_ | 0x6 | External Interrupt 2 |
| ----- | 0x8 - 0x17 | Reserved |
| TINT | 0x18 | Timer Interrupt |
| RINT | 0x1a | Serial Port Receive Interrupt |
| XINT | 0x1c | Serial Port Transmit Interrupt |
| TRAP | 0x1e | TRAP Instruction address |

## B.2.	TMS320C26 (Microcomputer/Bootloader mode) Interrupt Vector Map

The following interrupt vector map applies to the TMS320C26 in Microcomputer/Bootloader mode.

| Interrupt Name | Memory Location | Function |
| --- | --- | --- |
| ----- | 0xffa0 | Reserved |
| INT0_ | 0xffa2 | External Interrupt 0 |
| INT1_ | 0xffa4 | External Interrupt 1 |
| INT2_ | 0xffa6 | External Interrupt 2 |
| TINT | 0xffa8 | Timer Interrupt |
| RINT | 0xffaa | Serial Port Receive Interrupt |
| XINT | 0xffac | Serial Port Transmit Interrupt |
| TRAP | 0xffae | TRAP Instruction address |

## B.3.	TMS320C3x Interrupt Vector Map

The following interrupt vector map applies to all TMS320C3x devices except for the TMS320C31 in Microcomputer/Bootloader mode.

| Interrupt Name | Memory Location | Function |
| --- | --- | --- |
| RESET | 0x0 | External Reset |
| INT0_ | 0x1 | External Interrupt 0 |
| INT1_ | 0x2 | External Interrupt 1 |
| INT2_ | 0x3 | External Interrupt 2 |
| INT3_ | 0x4 | External Interrupt 3 |
| XINT0 | 0x5 | Serial Port 0 Transmit Int |
| RINT0 | 0x6 | Serial Port 0 Receive Int |
| XINT0 * | 0x7 | Serial Port 1Transmit Int |
| RINT0 * | 0x8 | Serial Port 1Receive Int |
| TINT0 | 0x9 | Timer 0 Interrupt |
| TINT1 | 0xa | Timer 1 Interrupt |
| DINT | 0xb | DMA Interrupt |
| ----- | 0xc - 0x1f | Reserved |
| TRAP 0 | 0x20 | TRAP 0 |
| TRAP 1 | 0x21 | TRAP 1 |
| . | . | . |
| . | . | . |
| . | . | . |
| TRAP 27 | 0x3b | TRAP 27 |
| ----- | 0x3c - 0x3f | Reserved |

* Reserved on the TMS320C31

## B.4.	TMS320C31 (Microcomputer/Bootloader mode) Interrupt Vector Map

The following interrupt vector map applies to the TMS320C31 in Microcomputer/Bootloader mode.

| Interrupt Name | Memory Location | Function |
| --- | --- | --- |
| INT0_ | 0x809fc1 | External Interrupt 0 |
| INT1_ | 0x809fc2 | External Interrupt 1 |
| INT2_ | 0x809fc3 | External Interrupt 2 |
| INT3_ | 0x809fc4 | External Interrupt 3 |
| XINT0 | 0x809fc5 | Serial Port 0 Transmit Int |
| RINT0 | 0x809fc6 | Serial Port 0 Receive Int |
| ----- | 0x809fc7 - 0x809fc8 | Reserved |
| TINT0 | 0x809fc9 | Timer 0 Interrupt |
| TINT1 | 0x809fca | Timer 1 Interrupt |
| DINT | 0x809fcb | DMA Interrupt |
| ----- | 0x809fcc - 0x809fdf | Reserved |
| TRAP 0 | 0x809fe0 | TRAP 0 |
| TRAP 1 | 0x809fe1 | TRAP 1 |
| . | . | . |
| . | . | . |
| . | . | . |
| TRAP 27 | 0x809ffb | TRAP 27 |
| ----- | 0x809ffc - 0x809fff | Reserved |

## B.5.	TMS320C4x Interrupt Vector Map

The following interrupt vector map applies to the TMS320C4x.

| Interrupt Name | Memory Location | Function |
| --- | --- | --- |
| ----- | IVTP + 0x0 | Reserved |
| NMI | IVTP + 0x1 | Non Maskable Interrupt |
| TINT0 | IVTP + 0x2 | Timer 0 Interrupt |
| IIOF0_ | IVTP + 0x3 | External Interrupt 0 |
| IIOF1_ | IVTP + 0x4 | External Interrupt 1 |
| IIOF2_ | IVTP + 0x5 | External Interrupt 2 |
| IIOF3_ | IVTP + 0x6 | External Interrupt 3 |
| ----- | IVTP + 0x7 to IVTP + 0xc | Reserved |
| ICFULL0 | IVTP + 0xd | Input Channel 0 Full |
| ICRDY0 | IVTP + 0xe | Input Channel 0 Ready |
| OCRDY0 | IVTP + 0xf | Output Channel 0 Ready |
| OCEMPTY0 | IVTP + 0x10 | Output Channel 0 Empty |
| ICFULL1 | IVTP + 0x11 | Input Channel 1 Full |
| ICRDY1 | IVTP + 0x12 | Input Channel 1 Ready |
| OCRDY1 | IVTP + 0x13 | Output Channel 1 Ready |
| OCEMPTY1 | IVTP + 0x14 | Output Channel 1 Empty |
| ICFULL2 | IVTP + 0x15 | Input Channel 2 Full |
| ICRDY2 | IVTP + 0x16 | Input Channel 2 Ready |
| OCRDY2 | IVTP + 0x17 | Output Channel 2 Ready |
| OCEMPTY2 | IVTP + 0x18 | Output Channel 2 Empty |
| ICFULL3 | IVTP + 0x19 | Input Channel 3 Full |
| ICRDY3 | IVTP + 0x1a | Input Channel 3 Ready |
| OCRDY3 | IVTP + 0x1b | Output Channel 3 Ready |
| OCEMPTY3 | IVTP + 0x1c | Output Channel 3 Empty |
| ICFULL4 | IVTP + 0x1d | Input Channel 4 Full |
| ICRDY4 | IVTP + 0x1e | Input Channel 4 Ready |
| OCRDY4 | IVTP + 0x1f | Output Channel 4 Ready |
| OCEMPTY4 | IVTP + 0x20 | Output Channel 4 Empty |
| ICFULL5 | IVTP + 0x21 | Input Channel 4 Full |
| ICRDY5 | IVTP + 0x22 | Input Channel 4 Ready |
| OCRDY5 | IVTP + 0x23 | Output Channel 4 Ready |
| OCEMPTY5 | IVTP + 0x24 | Output Channel 4 Empty |
| DMA INT0 | IVTP + 0x25 | DMA Channel 0 Interrupt |
| DMA INT1 | IVTP + 0x26 | DMA Channel 1 Interrupt |
| DMA INT2 | IVTP + 0x27 | DMA Channel 2 Interrupt |
| DMA INT3 | IVTP + 0x28 | DMA Channel 3 Interrupt |
| DMA INT4 | IVTP + 0x29 | DMA Channel 4 Interrupt |
| DMA INT5 | IVTP + 0x2a | DMA Channel 5 Interrupt |
| TINT1 | IVTP + 0x2b | Timer 1 Interrupt |
| ----- | IVTP + 0x2c to IVTP + 0x3f | Reserved |

## B.6.	TMS320C4x Trap Vector Map

The following Trap vector map applies to the TMS320C4x.

| Interrupt Name | Memory Location | Function |
| --- | --- | --- |
| TRAP 0 | TVTP + 0x0 | TRAP 0 |
| TRAP 1 | TVTP + 0x1 | TRAP 1 |
| . | . | . |
| . | . | . |
| . | . | . |
| TRAP 510 | TVTP + 0x1fe | TRAP 510 |
| TRAP 511 | TVTP + 0x1ff | TRAP 511 |

## B.7.	TMS320C5x Interrupt Vector Map

The following Trap vector map applies to the TMS320C5x devices.

| Interrupt Name | Memory Location | Function |
| --- | --- | --- |
| RS_ | 0x0 | Reset |
| INT1_ | IPTR + 0x2 | External Interrupt 1 |
| INT2_ | IPTR + 0x4 | External Interrupt 2 |
| INT3_ | IPTR + 0x6 | External Interrupt 3 |
| TINT | IPTR + 0x8 | Timer Interrupt |
| RINT | IPTR + 0xa | Serial Port Receive Int |
| XINT | IPTR + 0xc | Serial Port Transmit Int |
| TRNT * | IPTR + 0xe | TDM Port Receive Int |
| TXNT * | IPTR + 0x10 | TDM Port Transmit Int |
| INT4_ | IPTR + 0x12 | External Interrupt 4 |
| ----- | IPTR + 0x14 to IPTR + 0x21 | Reserved |
| TRAP | IPTR + 0x22 | TRAP Instruction Vector |
| NMI | IPTR + 0x24 | Non Maskable Interrupt |
| ----- | IPTR + 0x26 to IPTR + 0x3f | Reserved |

* Reserved on the TMS320C52

# C.	Code Examples

This appendix provides complete examples for setting up interrupt vectors in C for the processors discussed in this document.

## C.1.	TMS320C25 Complete Example

The following TMS320C25 example uses C entirely except to globally enable registers and build the vector table in memory. An asm() statement is used to enable the interrupts globally and an assembly language named section is used to create the vector table.

File: test.c

/**************************************************************************/

/*  TEST.C - Test program				*/

/**************************************************************************/

/**************************************************************************/

/*  MAIN - Main routine 					*/

/**************************************************************************/

void main(void)

{

initInts(); 		/* Initialize interrupts */

for(;;);			/* Replace with real code */

}

File: initvecs.c

/**************************************************************************/

/* INITVECS.C - Interrupt vector routines			*/

/**************************************************************************/

#define globalEnableInt()	asm("\t EINT");

/**************************************************************************/

/* initInts() - Initialize processor interrupt registers		*/

/**************************************************************************/

void initInts(void)

{

unsigned int *imr = (unsigned int *) 0x04;	/* IMR register */

*imr = 0x1; 				/* Enable INT0 */

globalEnableInt();

}

/**************************************************************************/

/* C_INT9 - Interrupt service routine			*/

/**************************************************************************/

#define dummyISR	c_int9			/* Rename ISR to correspond to TI */

/* naming conventions */

void dummyISR(void)

{

for(;;);					/* Replace with real ISR */

}

File: vecs.asm

***************************************************************************************

* VECS.ASM    - Reset and interrupt vector branch table for the C25	*

***************************************************************************************

.title		"vecs.asm"		;file name

.ref		_c_int0, _c_int9		;reference all interrupt

;vectors declared elsewhere

*****************************

* Reset and interrupt vectors

*****************************

.sect		"vectors"		;define named section

;for reset & interrupt vectors

b		_c_int0			;branch to reset vector

b		_c_int9			;branch to interrupt vector 1

b		_c_int9			;branch to interrupt vector 2

File: test.cmd

/***************************************************************************/

/* TEST.CMD - C25 linker command file			*/

/***************************************************************************/

-c

vecs.obj

initvecs.obj

test.obj

-mtest.map

-otest.out

-stack 0x400

-heap  0x400

-lrts25.lib

MEMORY

{

PAGE 0:	VECTORS:	origin = 00000h, length =  0002Ah

ROM:		origin = 00030h, length =  007CFh

P_RAM:	origin = 00800h, length =  02400h

EXT_PRGM:	origin = 02c00h, length =  05400h

PAGE 1:	REGS:		origin = 00000h, length =  00050h

I_O:		origin = 00050h, length =  00010h

RAMB2:	origin = 00060h, length =  00020h

RAMB1:	origin = 00300h, length =  00200h

INT_RAM:	origin = 00800h, length =  02400h

EXT_DATA:	origin = 08000h, length =  08000h

}

SECTIONS

{

"vectors":	{ } > VECTORS

.text:		{ } > P_RAM

.cinit:		{ } > P_RAM

.bss:		{ } > INT_RAM

.stack:		{ } > INT_RAM

}

## C.2.	TMS320C26 Complete Example

The following TMS320C26 is similar to the TMS320C25 example. For this example, a section named bootvecs is used to emulate the vector remapping that occurs due to the TMS320C26 boot loader.

File: test.c

/**************************************************************************/

/*  TEST.C - Test program				*/

/**************************************************************************/

/**************************************************************************/

/*  MAIN - Main routine 					*/

/**************************************************************************/

void main(void)

{

initInts(); 		/* Initialize interrupts */

for(;;);			/* Replace with real code */

}

File: initvecs.c

/**************************************************************************/

/* INITVECS.C - Interrupt vector routines			*/

/**************************************************************************/

#define globalEnableInt()    asm("\t EINT");

/**************************************************************************/

/* initInts() - Initialize processor interrupt registers		*/

/**************************************************************************/

void initInts(void)

{

unsigned int *imr = (unsigned int *) 0x04;	/* IMR register */

*imr = 0x1;				/* Enable INT0 */

globalEnableInt();

}

/**************************************************************************/

/* C_INT1 - Interrupt service routine			*/

/**************************************************************************/

#define dummyISR	c_int1	/* Rename ISR to correspond to TI */

/* naming conventions		 */

void dummyISR(void)

{

for(;;);			/* Replace with real ISR */

}

File: vecs.asm

**********************************************************************************

* VECS.ASM    - Reset and interrupt vector branch table for the C26   *

**********************************************************************************

.title		"vecs.asm"	;file name

.ref		_c_int1		;reference all interrupt

;vectors declared elsewhere

*****************************

* Reset and interrupt vectors

*****************************

.sect		"vectors"	;define named section

;for reset & interrupt vectors

.space		2*16		;reserved

b		_c_int1		;branch to interrupt vector 1

b		_c_int1		;branch to interrupt vector

File: boot.asm

********************************************************************

* Boot.asm - file to emulate boot loader vector remmapping

********************************************************************

.sect "bootvecs"

PROG:	.set	0FA00h			;Prog-Adress of B0

START	B	START,*,AR7		;Reset

B	PROG+2,*,AR0		;Interrupt 0

B	PROG+4,*,AR0		;Interrupt 1

B	PROG+6,*,AR0		;Interrupt 2

.space 16 * 16			;reserve 16 words

B	PROG+8,*,AR0		;Timer-Interrupt

B	PROG+10,*,AR0	;Serial-Port-Int.

B	PROG+12,*,AR0	;Serial-Port-Int.

B	PROG+14,*,AR0	;Software-Interrupt

File: test.cmd

/***************************************************************************/

/* TEST.CMD - C26 linker command file			*/

/***************************************************************************/

-c

vecs.obj

boot.obj

initvecs.obj

test.obj

-mtest.map

-otest.out

-stack 0x400

-heap  0x400

-lrts25.lib

MEMORY

{

PAGE 0 :	ROM:		origin = 00000h, length =  007FFh

P_RAM:	origin = 00800h, length =  02400h

VECTORS:	origin = 0FA00h, length =  00018h

EXT_PRGM:	origin = 02c00h, length =  05400h

PAGE 1 :	REGS:		origin = 00000h, length =  00050h

I_O:		origin = 00050h, length =  00010h

RAMB2:	origin = 00060h, length =  00020h

RAMB1:	origin = 00300h, length =  00200h

INT_RAM:	origin = 00800h, length =  02400h

EXT_DATA:	origin = 08000h, length =  08000h

}

SECTIONS

{

"bootvecs":	{ } > ROM

"vectors":	{ } > VECTORS

.text:		{ } > P_RAM

cinit:		{ } > P_RAM

.bss:		{ } > INT_RAM

.stack:		{ } > INT_RAM

}

## C.3.	TMS320C30 Complete Example

The following TMS320C3x example uses in line asm() statements to initialize the interrupt registers. An assembly language named section is used to create the vector table. Also, the TMS320C3x DMA is initialized to transfer data between two C arrays and cause an interrupt when complete. The arrays are defined such that they can be individualy located in the memory map in a different location from the .bss section such that the DMA and CPU will operate concurrently, that is, without a resource conflict. This example also illustrates how to install interrupts at run time by installing one additional vector at run time.

File: test.c

/**************************************************************************/

/*  TEST.C - Test program				*/

/**************************************************************************/

#include "vecs.h"

/**************************************************************************/

/*  MAIN - Main routine 					*/

/**************************************************************************/

void main(void)

{

initInts();			/* Enable interrupts */

dmaInit();			/* Setup DMA for transfer and int */

for(;;);				/* Replace with real code */

}

File: initvecs.c

/**************************************************************************/

/*INITVECS.C - Interrupt vector routines			*/

/**************************************************************************/

void c_int98(void);		/* function prototype */

/***************************************************************************/

/* initInts() - Initialize processor interrupt registers		*/

/***************************************************************************/

void initInts(void)

{

*((void (**) ()) 0x0b) = c_int98;	/* Install DMA interrupt vector */

/* at run-time */

asm("\t LDI \t 0h,IF");		/* Clear IF register */

asm("\t OR \t 403h,IE");		/* Enable INT0 & INT1 & DMA Ints */

asm("\t OR \t  2000h,ST");	/* enable GIE bit */

}

/**************************************************************************/

/*  C_INT99 - Interrupt service routine 			*/

/**************************************************************************/

#define dummyISR	c_int99		/* Rename ISR to correspond to TI */

/* naming conventions */

void dummyISR(void)

{

for(;;);				/* Replace with real ISR */

}

/**************************************************************************/

/*  C_INT98 - Interrupt service routine 			*/

/**************************************************************************/

#define dummyDMAISR	c_int98		/* Rename ISR to correspond to TI */

/* naming conventions */

void dummyDMAISR(void)

{

for(;;);				/* Replace with real ISR */

}

File: dma.c

/*******************************************************************************************/

/* DMA.C - 	Routine to setup the C3x DMA for a data transfer and interrupt */

/*		the CPU when done					*/

/*******************************************************************************************/

#include "vecs.h"

/**************************************************************************/

/* dmaInit() - DMA initialization routine			*/

/**************************************************************************/

void dmaInit(void)

{

/* Pointer to DMA */

volatile unsigned int *dma = (volatile unsigned int *) 0x808000;

/* Setup DMA to transfer data and set interrupt */

dma[4] = (unsigned int) sourceArray;

dma[6] = (unsigned int) destArray;

dma[8] = 5;

dma[0] = 0xc53;

}

File: datasrc.c

/*****************************************/

/* DATASRC.C - Source array	*/

/*****************************************/

int sourceArray[] = {1,2,3,4,5};

File: test.c

/*****************************************/

/* DATADST.C - Destination array */

/*****************************************/

int destArray[5];

File: vecs.h

/**************************************************************************/

/* VECS.H - Header file for interrupt vector program		*/

/**************************************************************************/

extern void initInts();

extern int  sourceArray[];

extern int  destArray[];

File: vecs.asm

******************************************************************************

* VECS.ASM - 	Reset and interrupt vector table for the 'C30 and   *

*		the 'C31 (Microprocessor Mode)			*

******************************************************************************

.title		"vecs.asm"	;file name

.ref		_c_int00, _c_int99	;reference all interrupt

*****************************

* Reset and interrupt vectors

*****************************

.sect		"vectors"	;define named section

.word		_c_int00	;reset vector

.word		_c_int99	;interrupt vector 1

.word		_c_int99	;interrupt vector 2

.end				;end of file

File: test.cmd

/***************************************************************************/

/* TEST.CMD - C30 linker command file			*/

/***************************************************************************/

-c

vecs.obj

dma.obj

initvecs.obj

test.obj

datasrc.obj

datadst.obj

-mtest.map

-otest.out

-stack 0x400

-heap  0x400

-lrts30.lib

/* SPECIFY THE SYSTEM MEMORY MAP */

MEMORY

{

VECS:	org = 0x0	len = 0x40

EXT0:	org = 0x100	len = 0x3f00	/* EXTERNAL MEMORY */

RAM0:	org = 0x809800	len = 0x400	/* RAM BLOCK 0 */

RAM1:	org = 0x809c00	len = 0x400	/* RAM BLOCK 1 */

}

/* SPECIFY THE SECTIONS ALLOCATION INTO MEMORY */

SECTIONS

{

"vectors"			> VECS

.bss:				> EXT0, block 0x10000

.text:				> EXT0

.cinit:				> EXT0

.stack:				> EXT0

source: {datasrc.obj(.bss)}	> RAM0

dest: {datadst.obj(.bss)}		> RAM1

}

## C.4.	TMS320C31 Complete Example

The following TMS320C31 example uses an assembly language named section to build the interrupt vector table and a C callable assembly language module to setup the interrupt registers. However, no parameters are passed to this routine.

File: test.c

/**************************************************************************/

/*  TEST.C - Test program				*/

/**************************************************************************/

#include "vecs.h"

/**************************************************************************/

/*  MAIN - Main routine 					*/

/**************************************************************************/

void main(void)

{

initInts();			/* Enable interrupts */

for(;;);				/* Replace with real code */

}

File: initvecs.c

/**************************************************************************/

/* INITVECS.C - Interrupt vector routines			*/

/**************************************************************************/

/**************************************************************************/

/*  C_INT99 - Interrupt service routine 			*/

/**************************************************************************/

#define dummyISR	c_int99		/* Rename ISR to correspond to TI */

/* naming conventions */

void dummyISR(void)

{

for(;;);				/* Replace with real ISR */

}

File: vecs.h

/**************************************************************************/

/* VECS.H - Header file for interrupt vector program		*/

/**************************************************************************/

extern void initInts();

File: initi.asm

************************************************************

* INIT.ASM - C-callable ASM routine to initialize interrupts

************************************************************

*function prototype: void initInts(void)

.file	"init.asm"

.globl	_initInts

FP	.set	AR3

_initInts:

PUSH	FP		;manage stack on entry

LDI	SP,FP

LDI	0h,IF		;clear IF register

OR	3h,IE		;enable INT0 & INT1

OR	2000h,ST	;set GIE bit to one to globally enable

;interrupts

LDI	*-FP(1),R1	;manage stack on exit

BD	R1

LDI	*FP,FP

NOP

SUBI	2,SP

***	B	R1		;branch occurs here

File: vecs.asm

**************************************************************************************

* VECS.ASM 	Interrupt vector branch table for the 'C31 (Bootloader Mode

**************************************************************************************

.title		"vecs.asm"		;file name

.ref		_c_int99		;reference all interrupt

;vectors declared elsewhere

*******************

* Interrupt vectors

*******************

.sect		"vectors"		;define named section

;interrupt branches

br		_c_int99		;branch to interrupt vector 1

br		_c_int99		;branch to interrupt vector 2

;note use of long-immediate

;addressing mode branch (br)

.end					;end of file

File: test.cmd

/***************************************************************************/

/* TEST.CMD - C31 linker command file			*/

/***************************************************************************/

-c

vecs.obj

init.obj

initvecs.obj

test.obj

-mtest.map

-otest.out

-stack 0x400

-heap  0x400

-lrts30.lib

/* SPECIFY THE SYSTEM MEMORY MAP */

MEMORY

{

SRAM:	org = 0x0		len = 0x100

EXT0:	org = 0x100		len = 0x3f00	/* EXTERNAL MEMORY */

RAM0:	org = 0x809800		len = 0x400	/* RAM BLOCK 0 */

RAM1:	org = 0x809c00		len = 0x3C1	/* RAM BLOCK 1 - 63 words */

VECS:	org = 0x809FC1	len = 0x3f

}

/* SPECIFY THE SECTIONS ALLOCATION INTO MEMORY */

SECTIONS

{

"vectors">	VECS

.text:	>	EXT0, block 0x10000

.bss:>		EXT0

.cinit:	>	EXT0

.const:	>	EXT0

.stack:	>	EXT0

.sysmem:>	EXT0

}

## C.5.	TMS320C40 Complete Example with the PRTS

The following TMS320C4x example uses the Parallel Runtime Support (PRTS) Library to setup the interrupt vectors and initialize the interrupt registers. When using the PRTS library, the vector map is created at runtime using the install_int_vector() function. The user selects the location of the vector map by placing the predefined named section ".vector" thought the linker command file. The IVTP register is initialized using the set_ivtp() function with the argument DEFAULT.

File: test.c

/**************************************************************************/

/*  TEST.C - Test program				*/

/**************************************************************************/

#include "vecs.h"

/**************************************************************************/

/*  MAIN - Main routine 					*/

/**************************************************************************/

void main(void)

{

initInts(); 		/* Initialize interrupt environment */

for(;;);

}

File: initvecs.c

/**************************************************************************/

/* INITVECS.C - Interrupt vector routines			*/

/**************************************************************************/

#include "vecs.h"

/**************************************************************************/

/* initInts() - Initialize processor interrupt registers		*/

/**************************************************************************/

void initInts(void)

{

set_ivtp(DEFAULT);		/* Set IVTP */

asm("\t LDI \t 0h,IIF");		/* clear the IIF reg */

set_iie(TIMER0);		/* Enable Timer0 int */

install_int_vector((void *) c_int99, 2);	/* Install timer interrupt */

asm("\t OR \t 2000h,ST");	/* enable GIE bit */

}

/**************************************************************************/

/*  C_INT99 - Interrupt service routine 			*/

/**************************************************************************/

#define dummyISR	c_int99		/* Rename ISR to correspond to TI */

/* naming conventions */

void dummyISR(void)

{

for(;;);				/* Replace with real ISR */

}

File: vecs.h

/**************************************************************************/

/* VECS.H - Header file for interrupt vector program		*/

/**************************************************************************/

#include <intpt40.h>

#include <timer40.h>

void c_int99(void);

File: test.cmd

/***************************************************************************/

/* TEST.CMD - C40 linker command file			*/

/***************************************************************************/

-c

test.obj

initvecs.obj

-mtest.map

-otest.out

-stack 0x400

-heap  0x400

-lprts40.lib

-lrts40.lib

/* SPECIFY THE SYSTEM MEMORY MAP */

MEMORY

{

VECS:		org = 0x000000	len = 0x40

RAM0:		org = 0x2FF800	len = 0x400		/* RAM BLOCK 0 */

RAM1:		org = 0x2FFC00	len = 0x400		/* RAM BLOCK 1 */

LOCAL:	org = 0x300000	len = 0x7D00000	/* LOCAL BUS */

GLOBAL:	org = 0x8000000	len = 0x8000000	/* GLOBAL BUS */

}

/* SPECIFY THE SECTIONS ALLOCATION INTO MEMORY */

SECTIONS

{

".vector"	> RAM0

.bss:		> LOCAL, block 0x10000

.text:		> LOCAL

.cinit:		> LOCAL

.const:		> LOCAL

.stack:		> LOCAL

.sysmem:	> RAM1

}

## C.6.	TMS320C40 Complete Example without the PRTS

The following TMS320C4x example uses linker assigned variables to pass the vector base address to a C callable assembly language module to initialize the interrupt vector table pointer. Other parameters are passed to the routine as well to initialize the other interrupt registers.  Here, two assembly language named sections are used to individually place the reset and interrupt vectors at different locations though the linker command file.

File: test.c

/**************************************************************************/

/*  TEST.C - Test program				*/

/**************************************************************************/

#include "vecs.h"

/**************************************************************************/

/*  MAIN - Main routine 					*/

/**************************************************************************/

void main(void)

{

initInts(&vecTable,0x1);		/* Initialize interrupt environment */

for(;;);				/* Replace with real code */

}

File: initvecs.c

/**************************************************************************/

/* INITVECS.C - Interrupt vector routines			*/

/**************************************************************************/

/**************************************************************************/

/*  C_INT99 - Interrupt service routine 			*/

/**************************************************************************/

#define dummyISR	c_int99		/* Rename ISR to correspond to TI */

/* naming conventions */

void dummyISR(void)

{

for(;;);				/* Replace with real ISR */

}

File: init.asm

************************************************************

* INIT.ASM - C-callable ASM routine to initialize interrupts

************************************************************

;function prototype void initInts(unsigned int **, unsigned int)

.file	"init.asm"

.globl	_initInts

FP	.set	AR3

_initInts:

PUSH	FP			;manage stack on entry

LDI	SP,FP

LDI	*-FP(2),R0		;get address of vector table

LDI	*-FP(3),IIE		;load int enable register

LDPE	R0,IVTP 		;store address of vector table

;in IVTP

LDI	0h,IIF			;clear the IIF register

OR	2000h,ST		;enable interrupts globally via GIE

;bit in Status register

LDI	*-FP(1),R1		;manage stack on exit

BD	R1

LDI	*FP,FP

NOP

SUBI	2,SP

***	B	R1			;branch occurs

.end

File: vecs.h

/**************************************************************************/

/* VECS.H Header file for interrupt vector program		*/

/**************************************************************************/

extern void initInts(unsigned int **, unsigned int);

extern unsigned int *vecTable;

File: vecs.asm

********************************************************************************

* VECS.ASM   - Reset and interrupt vector table for the 'C40. This    *

*	       example assumes that the reset vector and interrupt     *

*	       vectors will reside in different locations                         *

********************************************************************************

.title		"vecs.asm"	;file name

.def		IVECS		;IVECS defined in this module

.ref		_c_int00, _c_int99	;reference all interrupt

;vectors declared elsewhere

**************

* Reset vector

**************

.sect		"reset"		;define named section for

;reset vector

.word		_c_int00	;reset vector

*******************

* Interrupt vectors

*******************

.sect		"vectors"	;define named section

;for interrupt vectors

IVECS	.space		1		;one reserved location

.word		_c_int99	;interrupt vector 1

.word		_c_int99	;interrupt vector 2

File: test.cmd

/***************************************************************************/

/* TEST.CMD - C40 linker command file			*/

/***************************************************************************/

-c

init.obj

initvecs.obj

vecs.obj

test.obj

-mtest.map

-otest.out

-stack 0x400

-heap  0x400

-lrts40.lib

_vecTable = IVECS;		/* Set vecTable to point to vector table */

/* SPECIFY THE SYSTEM MEMORY MAP */

MEMORY

{

VECS:		org = 0x000000	len = 0x40

RAM0:		org = 0x2FF800	len = 0x400		/* RAM BLOCK 0 */

RAM1:		org = 0x2FFC00	len = 0x400		/* RAM BLOCK 1 */

LOCAL:	org = 0x300000	len = 0x7D00000	/* LOCAL BUS */

GLOBAL:	org = 0x8000000	len = 0x8000000	/* GLOBAL BUS */

}

/* SPECIFY THE SECTIONS ALLOCATION INTO MEMORY */

SECTIONS

{

"reset"		> VECS

"vectors"	> RAM1

.bss:		> LOCAL, block 0x10000

.text:		> LOCAL

.cinit:		> LOCAL

.const:		> LOCAL

.stack:		> RAM0

.sysmem:	> LOCAL

}

## C.7.	TMS320C50 Complete Example

The following TMS320C5x example uses linker assigned variables to pass the vector base address and register initial values to a C routine. This routine loads the interrupt vector table pointer and other interrupt registers using pointers to memory mapped registers. Here, two assembly language named sections are used to individually place the reset and interrupt vectors at different locations though the linker command file.

File: test.c

/**************************************************************************/

/*  TEST.C - Test program				*/

/**************************************************************************/

#include "vecs.h"

/**************************************************************************/

/*  MAIN - Main routine 					*/

/**************************************************************************/

void main(void)

{

initInts(&vecTable,0x3);		/* Enable interrupts INT0 and INT1 */

for(;;);				/* Replace with real code */

}

File: initvecs.c

/**************************************************************************/

/* INITVECS.C - Interrupt vector routines			*/

/**************************************************************************/

/**************************************************************************/

/* initInts() - Initialize processor interrupt registers		*/

/**************************************************************************/

void initInts(unsigned int **vecTable, unsigned int imrValue)

{

unsigned int *imr  = (unsigned int *) 0x04;	/* IMR register */

unsigned int *ifr  = (unsigned int *) 0x06;		/* IFR register */

unsigned int *pmst = (unsigned int *) 0x07;	/* PMST register */

*pmst |= (unsigned int) vecTable;

*ifr   = 0xffff;				/* Clear IFR register */

*imr  |= imrValue;			/* Load IMR register */

asm("\t CLRC\t INTM");			/* Clear interrupt mask bit */

}

/**************************************************************************/

/*  C_INT3 - Interrupt service routine 			*/

/**************************************************************************/

#define dummyISR	c_int3			/* Rename ISR to correspond to TI */

/* naming conventions */

void dummyISR(void)

{

for(;;);				/* Replace with real ISR */

}

File: vecs.h

/**************************************************************************/

/* VECS.H - Header file for interrupt vector program		*/

/**************************************************************************/

extern void initInts(unsigned int **, unsigned int);

extern unsigned int *vecTable;

File: vecs.asm

************************************************************************

* VECS.ASM   - Reset and interrupt vector table for the 'C50. *

************************************************************************

.title		"vecs.asm"	;file name

.def		IVECS		;IVECS defined in this module

.ref		_c_int0, _c_int3	;reference all interrupt

;vectors declared elsewhere

**************

* Reset vector

**************

.sect		"reset"		;define named section for

;reset vector

b		_c_int0 		;reset vector

*******************

* Interrupt vectors

*******************

.sect		"vectors"	;define named section

;for interrupt vectors

IVECS	.space		2*16		;one reserved location

b		_c_int3		;interrupt vector 1

b		_c_int3		;interrupt vector 2

File: test.cmd

/***************************************************************************/

/* TEST.CMD - C50 linker command file			*/

/***************************************************************************/

-c

vecs.obj

initvecs.obj

test.obj

-mtest.map

-otest.out

-stack 0x400

-heap  0x400

-lrts50.lib

_vecTable = IVECS;	/* Set vecTable to point to vector table */

MEMORY

{

PAGE 0:	VECTORS:	origin = 00000h, length =  0002fh

ROM:		origin = 00030h, length =  007CFh

P_RAM:	origin = 00800h, length =  023FFh

EXT_PRGM:	origin = 02c00h, length =  0D1FFh

PAGE 1:	REGS:		origin = 00000h, length =  00050h

I_O:		origin = 00050h, length =  00010h

RAMB2:	origin = 00060h, length =  00020h

RAMB0_D:	origin = 00100h, length =  00200h

RAMB1:	origin = 00300h, length =  00200h

INT_RAM:	origin = 00800h, length =  023FFh

EXT_DATA:	origin = 02C00h, length =  0FFFFh

}

SECTIONS

{

"reset"		> VECTORS

"vectors"	> P_RAM

.text :		> ROM

.cinit :		> ROM

.bss :		> RAMB0_D

.stack :		> INT_RAM

}