MOD-EP1(-EP4)   E(E)PROM PROGRAMMER

DEVICES PROGRAMMED

§ EPROM		2716 - 27C080	(8bit)

§ FLASH		28F256 - 28F020

§ PAGE 		27C53, 27C011

§ EEPROM	2804 - 2864, 29C256 - 29C010

DEVICES PROGRAMMED (ADAPTER)

ü1: RM-875X (40-pin DIP)

87C51, 87C52, 87C54, 8751BH, 8752BH, 8751H,

87C51FA/B/C, 87C521, 87C524, 87C528, 87C654

ü2: RM-16BIT(40-pin DIP)

27C210 - 27C240	(16bit)

27C1024 - 27C4096	(16bit)

541024 - 544096	(16bit)

571024 - 574096	(16bit)

27C102 - 27C402	(16bit)

ü3: RM-PIC16C5X

PIC16C54, 16C55, 16C56, 16C57

ü4: PA751

87C751, 87C752

üCE: Contact E.E. Tools

§ PLCC ADAPTERS:

PA44-40A: Pin 1, 12, 23, 34 skipped,  87C5x family PLCC

PA44-40B: Pin 1, 13, 23, 33 skipped,  27210-274096 PLCC

PA32-32: All pins connected for 27010 - 27080 PLCC

PA32-28: Pin 1, 12, 17, 26 skipped for 2764 - 27512 PLCC

KEY FEATURES

§ Auto search device select function supports.

§ Device Operations: Read, Blank check, Program, Verify, Checksum, Data compare, Security, Auto programming.

§ Extensive on line F1 help system provides text and graphics.

§ Auto Programming for production programming.

§ Built-in editor for both buffer date and test vectors.

§ Support Intel Hex, Motorola S Records, Tekhex, and binary file. formats with Load, Edit, and Save commands.

§ Distribution of 16- and 32- bit data into 8-bit portions.

§ 32-pin ZIF(Zero Insertion Force) socket accepts both 300mil

and 600 mil DIP devices up to 32-pin EPROMs.

SYSTEM REQUIREMENT

§ TYPE: PC, XT, AT, 386, 486, Pentium, or compatibles.

§ A hard disk drive is recommended for software installation.

§ RAM size: 300K.

§ I/O PORT: 64 byte consecutive address(200 - 2E0 hex).

§ OS: MS-DOS 2.11 or greater.

BASIC TECHNICAL SPECIFICATION

§ Dimension:

Programmer: 5.7”(L) x 3.3”(W)  x 1.3”(H)

Board: 4.8” x 4.3”

Shipping Weight: 2 lbs(MOD-EP1), 3 lbs(MOD-EP4)

§ Host Interface:

MOD-EP1(EP4) use ISA bus plug-in card,

I/O port base address(200 - 2E0hex)

§ Socket: Textool 32-pin ZIF standard.

§ MOD-EP1(-EP4) package contains:

• 32-pin ZIF socket(MOD-EP1) programming module.

• 32-pin ZIF 4-socket(MOD-EP4) programming module.

• PC plug-in card.

• 3 feet long 40-pin ribbon cable.

• Installation diskette and manual.